WOLDIA UNIVERSITY

INSTITUTE OF TECHNOLOGY

SCHOOL OF ELECTRICAL AND COMPUTER ENGINEERING

COMPUTER ENGINEERING STREAM

TITLE: DESIGN AND IMPLEMENTATION OF SMART SIGNAGE SYSTEM FOR THE PURPOSE OF ORGANIZATIONAL INFORMATION DISCRIMINATION

PREPARED BY:

GROUP NAME						ID NO

- DAWIT BERHAN						WDU1304696

- CHARAKA WAKSHUM					WDU1300748

- ARSEMA DANIEL						WDU1300391

ACADEMIC ADVISOR ALI Y. (MSc.)

SUBMITTED TO: ECE Department

SUBMITTED DATE 20/09/2018 E.C.

WOLDIA, ETHIOPIA

# Declaration

Here by, declare that the work entitles design and implementation of smart signage system for the purpose of organizational information discrimination is our original work, we have not copied from any other student’s work or from any other source except where due reference or acknowledgment is made explicitly in the text, nor has any part been written for us by another person.

Student’s Name							Signature

Dawit Berhan								__________

Chereka Wakshum							__________

Arsema Daniel							__________

Advisor’s Approval

The project has been submitted for examination with approval as a university advisor.

Advisor’s Name			Signature			Date

Mr. Ali Yimam (MSc.)		__________			__________

Examiner Committee		Signature			Date

1. _________________		___________			___________

2. _________________		___________			___________

3. _________________		___________			___________

# Acknowledgements

We would like to express our sincere gratitude to our advisor, Mr. Ali Y, for the guidance, encouragement, and technical insight provided throughout this capstone project. His feedback at every milestone was instrumental in shaping both the architecture and the presentation of our work.

We thank the Department of Electrical and Computer Engineering for providing the laboratory facilities and equipment that enabled us to assemble and test the sensor hardware prototype. We also acknowledge our fellow students and peers whose discussions and suggestions refined our design decisions.

This project was completed collaboratively by three final-year students working as a group. All system design, hardware assembly, software development, testing, and documentation were shared responsibilities, with each member contributing to every subsystem.

# Abstract

Modern organizations rely on digital signage for information dissemination, yet existing solutions suffer from four critical gaps: displays operate at fixed brightness regardless of ambient conditions, each node is managed independently without unified control, IoT devices receive content without authentication, and commercial platforms impose prohibitive licensing costs exceeding $1,000 per display annually. This capstone project addresses these gaps through the design and implementation of a Smart Digital Signage System that integrates environmental sensing, role-based content management, real-time device control, live stream distribution, and security-hardened network infrastructure.

We assembled an Arduino Mega 2560 sensor bridge with three HC-SR04 ultrasonic distance sensors, a light-dependent resistor, a potentiometer, and an emergency push button on a solderless breadboard. Two laptops running Debian 13 live USB sessions simulated Raspberry Pi edge nodes, with their screens serving as the digital signage displays. The Arduino transmitted sensor data over USB serial at 9600 baud to the Debian nodes, enabling real-time adaptive brightness control via the operating system’s brightness API, occupancy detection for contextual content scheduling, and an emergency broadcast system triggered by a hardware push button.

The backend was implemented in Node.js with Express.js, using Prisma ORM with PostgreSQL for data persistence, Socket.IO for real-time device command and control, and FFmpeg with Sharp for media processing. The frontend was built in React 19 with Vite, featuring role-based dashboards for administrators and content creators, a public feed with AI-assisted Q&A, and a dual-designer architecture supporting both visual (Fabric.js) and textual (Markdown + KaTeX) content creation.

Validation comprised 30+ backend integration tests using Jest and Supertest, 25 frontend end-to-end tests using Playwright producing 55 screenshots, and hardware verification confirming sensor data accuracy, serial communication stability, and end-to-end brightness adaptation. Our cost analysis projects a 60% total cost of ownership reduction compared to commercial alternatives over a five-year deployment horizon.

Keywords: digital signage, IoT, environmental sensing, adaptive brightness, RBAC, Raspberry Pi, Arduino, Socket.IO, live streaming, campus network, emergency broadcast, full-stack development

- Table of Contents

Declaration	i

Acknowledgements	ii

Abstract	iii

List of Figures	xi

List of Tables	xv

Acronyms and Abbreviations	xvi

Chapter 1	1

1	Introduction	1

1.1	Background and Motivation	1

1.1.1	Evolution of Campus Communication	1

1.1.2	Limitations of Current Commercial and Open-Source Digital Signage	2

1.1.3	The Convergence Opportunity	3

1.2	Problem Statement	4

1.3	Objective	5

1.3.1	General Objective	5

1.3.2	Specific Objectives	5

1.4	Scope and Delimitations	6

1.4.1	In scope	6

1.4.2	Out of scope	6

1.4.3	Justification	7

1.5	Significance and Contributions	7

1.5.1	Why This Matters	7

1.5.2	Novel Contributions	7

1.5.3	Who Benefits	8

1.6	Report Structure	8

Chapter 2	10

2	Literature Review and Related Work	10

2.1	Digital Signage Platforms	10

2.1.1	Open-Source Solutions	10

2.1.2	Commercial Enterprise Systems	11

2.1.3	Research Gaps Identified	12

2.2	IoT and Edge Computing for Building Automation	13

2.2.1	Raspberry Pi as Edge Node	13

2.2.2	Arduino-Based Sensor Acquisition	13

2.2.3	Serial Communication Protocols	14

2.3	Power Management in Display Systems	15

2.3.1	Ambient Light Sensors for Brightness Adaptation	15

2.3.2	Psychophysical Basis: The Weber-Fechner Law	15

2.3.3	CEC and DDC/CI Protocols for Display Control	17

2.4	Live Streaming in Digital Signage	17

2.4.1	RTMP Ingest and HLS Distribution	17

2.4.2	Existing Platform Streaming Capabilities	18

2.5	Multi-Tenancy and Role-Based Access Control	18

2.5.1	IoT Multi-Tenancy Literature	18

2.5.2	Campus Network Security for IoT	19

2.6	Research Gaps Summary	19

Chapter 3	21

3	Methodology	21

3.1	Research Approach	21

3.2	System Requirements	22

3.2.1	Functional Requirements	22

3.2.2	Non-Functional Requirements	23

3.3	High-Level Architecture	24

3.3.1	End-to-End Data Flow	25

3.3.2	Component Count and Scale	26

3.4	Development and Testing Methodology	27

3.4.1	Iterative Development Process	27

3.4.2	Testing Strategy	27

3.4.3	Development Tooling	28

Chapter 4	30

4	System Design and Implementation	30

4.1	Hardware Implementation	30

4.1.1	Component Selection and Specifications	30

4.1.2	Physical Assembly and Wiring	31

4.1.3	Sensor Fusion and Preprocessing	33

4.1.4	Adaptive Brightness Algorithm	34

4.1.5	Simulated Edge Node Software	35

4.1.6	Systemd Service Configuration	35

4.2	Network and Infrastructure Design	36

4.2.1	Subnet Design	36

4.2.2	Layer 3 Isolation with Dual-NIC Server	37

4.2.3	Core Services	38

4.2.4	Bandwidth Engineering	39

4.2.5	Failure Modes and Resilience	39

4.2.6	Security Hardening at the Network Layer	40

4.3	Backend System Implementation	40

4.3.1	Layered Architecture	40

4.3.2	Database Design	41

4.3.3	Authentication and Authorization	42

4.3.4	User and Group Management	43

4.3.5	Content Lifecycle Management	44

4.3.6	Device Management and Control Locks	45

4.3.7	Signage Deployment Engine	46

4.3.8	Media Processing Pipeline	47

4.3.9	Live Stream Relay	48

4.3.10	Emergency Broadcast System	48

4.3.11	Socket.IO Real-Time Bus	49

4.3.12	Fault Tolerance and Race Condition Handling	50

4.3.13	AI-Assisted Public Engagement	50

4.4	Frontend and User Interface	50

4.4.1	Admin Dashboard	51

4.4.2	Creator Dashboard	53

4.4.3	Public Feed and Viewer Interface	57

4.4.4	AI Q&A Widget	58

4.4.5	Emergency Handling UI	59

4.4.6	State Management	60

4.5	Dual Player Architecture	60

4.6	Security Implementation	61

4.6.1	Threat Model	61

4.6.2	Vulnerabilities Addressed	61

4.6.3	Network Hardening	64

4.6.4	Remediation Summary	64

Chapter 5	65

5	Results and Validation	65

5.1	Functional Testing	65

5.1.1	Backend Integration Testing	65

5.1.2	Frontend End-to-End Testing	66

5.1.3	Test Infrastructure	67

5.2	Hardware Verification	68

5.2.1	Assembly Checklist	68

5.2.2	Power Measurement	68

5.2.3	Serial Communication Verification	68

5.2.4	End-to-End Brightness Adaptation Test	69

5.3	Network Verification	70

5.3.1	Connectivity Tests	70

5.3.2	Security Verification	70

5.4	Objectives Achievement	71

5.5	Performance and Cost Analysis	72

5.5.1	Power Consumption by Component	72

5.5.2	Campus-Wide Energy Scenarios	72

5.5.3	Campus-Wide Power Distribution	74

5.5.4	Cost Analysis	74

Chapter 6	76

6	Discussion	76

6.1	How the Problems Are Solved	76

6.1.1	Problem P1 - Energy Waste from Fixed-Brightness Operation	76

6.1.2	Problem P2 - Disconnected, Unmanaged Display Nodes	76

6.1.3	Problem P3 - Security Gap in Unauthenticated IoT Communication	77

6.1.4	Problem P4 - High Total Cost of Ownership	77

6.2	Implementation Readiness	78

6.2.1	Production-Ready Components	78

6.2.2	Components Requiring Field Validation	78

6.3	Limitations and Assumptions	79

6.4	Comparison with Related Work	79

Chapter 7	82

7	Conclusion and Future Work	82

7.1	Conclusion	82

7.2	Future Work	83

7.2.1	Immediate Priorities (0–6 months)	83

7.2.2	Medium-Term Enhancements (6–18 months)	83

7.2.3	Research Directions (18+ months)	84

References	86

Appendices	95

A.	Appendix A - Database Entity-Relationship Diagram	95

B.	Appendix B - Complete REST API Endpoint Reference	95

C.	Appendix C - Socket.IO Event Protocol Reference	97

D.	Appendix D - Arduino Firmware (sensors.ino)	98

E.	Appendix E - Raspberry Pi Agent Configuration (config.py)	99

F.	Appendix F - Prisma Schema Excerpt (Key Models)	100

G.	Appendix G - Network Configuration Files	100

H.	Appendix H - Frontend Component Hierarchy	102

I.	Appendix I - Systemd Service Units	102

J.	Appendix J - Security Vulnerability Detail Cards	103

K.	Appendix K - Risk Register	103

L.	Appendix L - Glossary of Terms	104

# List of Figures

Figure 1-1Traditional campus bulletin board with paper notices, illustrating the limitations of static communication: manual updates, no remote management, and no targeting by audience or location.	2

Figure 1-2 Commercial digital signage installation showing a fixed-brightness LCD display, illustrating energy waste and visual glare in low ambient light conditions.	3

Figure 1-3 High level view of the smart signage system, with network, device and users overview	8

Figure 2-1 Logarithmic brightness adaptation curve based on the Weber-Fechner law (solid) compared with linear mapping (dashed), showing the energy savings zone at low ambient light levels.	16

Figure 3-1High-level system block diagram showing the end-to-end data flow from sensors through Arduino, Debian edge node, Socket.IO, Node.js backend, PostgreSQL database, and React frontend.	24

Figure 3-2 Complete system topology showing 130+ Debian edge nodes, central Ubuntu server with dual-NIC Layer 3 isolation, core and edge switches, and campus subnet 10.20.0.0/22.	25

Figure 3-3 Terminal capture of the Git log showing the last 20 commits with Conventional Commit prefixes (feat, fix, docs, test, refactor), demonstrating our structured commit history over the development period.	29

Figure 4-1 Photograph of our assembled prototype showing the Arduino Mega 2560 on a solderless breadboard with three HC-SR04 sensors, connected via USB to a Debian 13 laptop.	32

Figure 4-2 Sensor wiring schematic showing Arduino Mega pin assignments for HC-SR04 (D22–D27), LDR (A0), potentiometer (A1), and emergency button (D2).	32

Figure 4-3 Photograph of the sensor array mounted near the laptop screen, showing the three HC-SR04 sensors positioned for left, center, and right coverage.	34

Figure 4-4 Screenshot of the Arduino IDE Serial Monitor at 9600 baud, showing five consecutive SENSOR: packets with motion, brightness, rain, and emergency values.	34

Figure 4-5 Screenshot of the systemd status output showing socket-signage.service as active (running), confirming automatic startup on boot.	36

Figure 4-6 Network topology diagram showing the dual-NIC Ubuntu server, core switches, edge switches, and Debian nodes in the 10.20.0.0/22 signage subnet.	38

Figure 4-7  Firewall rules diagram showing NIC-1 campus-facing rules (SSH/HTTP/HTTPS/NTP) and NIC-2 signage-facing rules (API/RTMP/HLS/DHCP/DNS).	40

Figure 4-8 Backend layered architecture diagram showing the data flow from Routes through Middleware, Services, Repositories, Prisma ORM, to PostgreSQL.	41

Figure 4-9 Authentication flow sequence diagram showing JWT login for users and device token assignment for IoT nodes.	43

Figure 4-10 RBAC permission matrix diagram showing the mapping of roles (admin, creator, viewer, public) to system capabilities.	44

Figure 4-11 Control lock acquisition flow sequence diagram showing Administrator A acquiring a lock on Device 3, and Creator B receiving a 423 Locked response.	46

Figure 4-12 Media processing pipeline flowchart showing image processing (Sharp), video processing (FFmpeg), and document text extraction (mammoth/pdf-parse-fork) paths.	47

Figure 4-13 Stream relay pipeline diagram showing OBS → RTMP → FFmpeg → HLS segments → nginx → Anthias/MPV playback.	48

Figure 4-14 Emergency broadcast state machine diagram showing transitions between Normal, Emergency, Disconnected, SecurityRisk, and BreakingNews states.	49

Figure 4-15 Login page with empty username and password fields, campus branding visible; Sign In button disabled until both fields are filled.	51

Figure 4-16 Admin Groups page listing all campus groups with display state, member count, and assigned devices.	52

Figure 4-17 Admin Users page showing all registered accounts with roles, groups, and permissions	52

Figure 4-18 Admin Devices page showing the registered hardware fleet with online/offline status, register form, and settings inspector.	53

Figure 4-19 Creator Posts page with the new-post editor on the left and the post list with filters on the right.	54

Figure 4-20 Image cropping interface with aspect ratio locking to 16:9 for signage compatibility, drag and zoom controls visible.	54

Figure 4-21 Video trimming slider used to extract specific clips before attaching to a post, start and end handles visible.	55

Figure 4-22 Creator Visual Designer with an empty Fabric.js canvas and toolbar on the top for adding text, shapes, and images.	55

Figure 4-23 Designer in Markdown mode with empty editor on the top and live-preview panel on the buttom, supporting KaTeX math notation.	56

Figure 4-24 Signage Publish page listing publishable posts on the left and target devices on the right, with duration and priority controls.	56

Figure 4-25 Live Streams page with the list of existing streams showing source type and status, plus the creation form.	57

Figure 4-26 Public feed showing all campus announcements sorted by priority and recency, no login required.	58

Figure 4-27 AI chat window opened on a post detail page, ready for user questions grounded on post text and attachments.	59

Figure 4-28 Groups page showing the Emergency Broadcast group in EMERGENCY state with red indicator and inline state controls.	59

Figure 4-29 Attack surface diagram showing trust boundaries: Campus LAN → Server DMZ → Database Zone → Signage LAN, with identified attack vectors at each layer.	61

Figure 4-30 Screenshot of the Socket.IO authentication code showing device token verification in the connection handshake handler.	63

Figure 5-1 Screenshot of the Jest test output showing all backend integration tests passing with green checkmarks and the final coverage summary.	66

Figure 5-2 Screenshot of the Playwright HTML test report showing all tests passed, with the test tree and execution times.	67

Figure 5-3 Screenshot of the Arduino IDE Serial Monitor at 9600 baud showing continuous SENSOR: packets with motion, brightness, rain, and emergency values during a 30-minute test.	69

Figure 5-4 Terminal screenshot showing successful ping, chronyc tracking, and API health curl responses from a Debian node.	70

Figure 5-5 24-hour campus power profile comparing fixed brightness (baseline), adaptive brightness, and conservative adaptive scenarios across 130 nodes.	73

Figure 5-6 Campus-wide power distribution pie chart showing displays (82%), compute (10%), network infrastructure (5%), and server (3%).	74

Figure 5-7 Five-year TCO comparison bar chart showing our system ($118,000) versus Yodeck ($218,000), Rise Vision ($187,000), NoviSign ($390,000), and ScreenCloud ($260,000).	75

Figure 7-1 Future multi-campus topology concept showing three campuses connected via WAN to a central management hub, with local servers at each campus for content autonomy and shared emergency broadcast capability.	84

Figure 0-1 Prisma Schema Entity-Relationship Diagram.	95

# List of Tables

Table 1-1 Specific objectives and their validation evidence.	5

Table 2-1 Commercial digital signage platform comparison.	12

Table 2-2 Serial protocol comparison for the Pi-Arduino data link.	14

Table 2-3 Research gaps and corresponding project objectives.	19

Table 3-1 Functional requirements.	22

Table 3-2 Non-functional requirements.	23

Table 3-3 Campus-wide component inventory.	26

Table 4-1 Component specifications and Arduino pin assignments.	30

Table 4-2 Structured IP allocation within 10.20.0.0/22.	36

Table 4-3 Bandwidth analysis by operational scenario.	39

Table 4-4 Role capabilities matrix.	43

Table 4-5 Stream types and relay architecture.	48

Table 4-6 Vulnerability inventory and remediation status.	61

Table 5-1 Objectives achievement summary	71

Table 5-2 Power consumption by system state (calculated from datasheets).	72

Table 5-3 Per-node bill of materials.	74

Table 6-1 Research gap mapping and comparison with related work.	80

# Acronyms and Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| AI | Artificial Intelligence |
| API | Application Programming Interface |
| BOM | Bill of Materials |
| CEC | Consumer Electronics Control |
| CI/CD | Continuous Integration / Continuous Deployment |
| CMS | Content Management System |
| CRUD | Create, Read, Update, Delete |
| CSS | Cascading Style Sheets |
| DDC/CI | Display Data Channel Command Interface |
| DHCP | Dynamic Host Configuration Protocol |
| DNS | Domain Name System |
| DOM | Document Object Model |
| E2E | End-to-End |
| FFmpeg | Fast Forward Moving Picture Experts Group (media framework) |
| GPIO | General-Purpose Input/Output |
| GPT | Generative Pre-trained Transformer |
| GPU | Graphics Processing Unit |
| GUI | Graphical User Interface |
| HDMI | High-Definition Multimedia Interface |
| HLS | HTTP Live Streaming |
| HTML | HyperText Markup Language |
| HTTP | Hypertext Transfer Protocol |
| HTTPS | Hypertext Transfer Protocol Secure |
| I/O | Input/Output |
| I2C | Inter-Integrated Circuit |
| IoT | Internet of Things |
| IP | Internet Protocol |
| JPEG | Joint Photographic Experts Group |
| JS | JavaScript |
| JSON | JavaScript Object Notation |
| JWT | JSON Web Token |
| LAN | Local Area Network |
| LDR | Light-Dependent Resistor |
| LED | Light-Emitting Diode |
| MP4 | MPEG-4 Part 14 |
| MPV | Media Player (open-source video player) |
| NTP | Network Time Protocol |
| npm | Node Package Manager |
| ORM | Object-Relational Mapping |
| OS | Operating System |
| PDF | Portable Document Format |
| PNG | Portable Network Graphics |
| PWM | Pulse-Width Modulation |
| RBAC | Role-Based Access Control |
| REST | Representational State Transfer |
| ROM | Read-Only Memory |
| RTMP | Real-Time Messaging Protocol |
| RTSP | Real-Time Streaming Protocol |
| RX | Receive |
| SLA | Service Level Agreement |
| SoC | System on Chip |
| SPI | Serial Peripheral Interface |
| SQL | Structured Query Language |
| SSL | Secure Sockets Layer |
| TCO | Total Cost of Ownership |
| TLS | Transport Layer Security |
| TX | Transmit |
| UART | Universal Asynchronous Receiver/Transmitter |
| UI | User Interface |
| UPS | Uninterruptible Power Supply |
| URL | Uniform Resource Locator |
| USB | Universal Serial Bus |
| UX | User Experience |
| WAN | Wide Area Network |
| WebP | Web Picture format |
| XML | Extensible Markup Language |

# Chapter 1

# Introduction

## Background and Motivation

### Evolution of Campus Communication

The transition from paper-based bulletin boards to electronic displays represents one of the most visible manifestations of digital transformation in educational and institutional environments. For decades, universities, hospitals, and corporate campuses relied on static printed notices posted on physical boards - a medium that was inexpensive to deploy but fundamentally limited in reach, timeliness, and adaptability [1]. Messages required manual replacement, could not be updated remotely, offered no targeting by audience or location, and provided no mechanism for emergency broadcast.

The advent of liquid-crystal and light-emitting diode (LED) display panels, combined with falling hardware costs and ubiquitous network connectivity, has enabled a generational shift toward digital signage. Modern campuses now deploy dozens to hundreds of electronic displays across buildings, corridors, courtyards, and parking areas, showing everything from class schedules and event announcements to cafeteria menus and emergency alerts [2]. These displays are typically managed through networked media players that download content from a central server and render it on high-definition screens ranging from 32 inches in elevators to 85 inches in atriums.

Despite this technological progress, the operational model of most campus digital signage systems remains strikingly static. Displays are turned on in the morning and off in the evening, running at a fixed brightness level calibrated for worst-case viewing conditions. Content schedules are set manually and updated infrequently. Each display or small cluster operates as an isolated unit with limited awareness of its physical environment or the presence of viewers. The result is a system that consumes significant electrical energy while delivering messages that may be irrelevant to the current audience or even invisible due to glare from changing ambient light [3].

Figure 1-1Traditional campus bulletin board with paper notices, illustrating the limitations of static communication: manual updates, no remote management, and no targeting by audience or location.

### Limitations of Current Commercial and Open-Source Digital Signage

The digital signage market offers two primary categories of solutions: proprietary commercial platforms and open-source alternatives. Commercial systems such as Screenly, Xibo, and Scala provide polished management dashboards, cloud hosting, and technical support, but impose recurring licensing fees that can exceed $1,000 per display per year [4]. For a 130-display campus, this translates to $130,000 in annual licensing alone - before hardware, installation, or energy costs. Furthermore, commercial platforms typically offer limited or no integration with environmental sensors, treating each display as a passive playback endpoint rather than an intelligent node [5].

Open-source alternatives, notably Screenly OSE and its community fork Anthias, eliminate licensing fees but introduce their own constraints. These platforms were designed primarily for simple playlist management: a sequence of images and videos played in rotation [6]. They lack native support for real-time data integration, environmental responsiveness, or role-based content governance. Administrative access is typically binary - either full control or none - making them unsuitable for multi-department campuses where different groups require scoped content management authority [7].

Neither commercial nor open-source platforms adequately address three critical operational requirements we identified through our requirements analysis:

Environmental responsiveness: No major platform automatically adjusts display brightness based on ambient light, despite the well-documented relationship between backlight intensity and power consumption [8]. Displays running at full brightness in dimly lit corridors or at night waste electricity and may cause visual discomfort.

Occupancy awareness: Content plays regardless of whether anyone is present. A display in an empty lecture hall at midnight consumes the same power as one in a crowded student center at noon. This absence of viewer detection eliminates opportunities for power savings and contextual content adaptation [9].

Unified emergency management: During campus emergencies - fire, severe weather, security incidents - individual displays cannot be overridden with critical information without manually interrupting their normal operation. The lack of a centralized emergency broadcast channel represents a genuine safety gap [10].

Figure 1-2 Commercial digital signage installation showing a fixed-brightness LCD display, illustrating energy waste and visual glare in low ambient light conditions.

### The Convergence Opportunity

The emergence of low-cost microcontrollers, single-board computers, and high-bandwidth campus networks creates an unprecedented opportunity to reimagine digital signage as an intelligent building automation system rather than a passive media playback infrastructure. The Arduino Mega 2560 ($25), Raspberry Pi 4B ($55), and HC-SR04 ultrasonic sensor ($2) make it economically feasible to instrument every display node with environmental sensing [11]. The proliferation of 1 Gbps wired Ethernet and managed switches enables real-time data collection from hundreds of nodes without congestion [12]. Modern JavaScript frameworks (React, Node.js) and containerization (Docker) reduce the engineering effort required to build production-grade management platforms [13].

This convergence suggests that a unified system - one that combines environmental sensing, intelligent display control, role-based content management, and secure network infrastructure - is not merely desirable but technically and economically achievable. Such a system would represent a genuine contribution to both the electrical engineering literature (sensor integration, power analysis, control systems) and the computer engineering literature (network design, full-stack development, security hardening) [14].

## Problem Statement

This project addresses the design and implementation of a Smart Digital Signage System that overcomes the three critical gaps identified in Section 1.1.2 through an integrated hardware–software architecture spanning sensor instrumentation, embedded processing, network infrastructure, backend services, frontend dashboards, and display control.

The specific problems we addressed are:

P1 - Energy waste from fixed-brightness operation: Digital displays on modern campuses consume 30–150 W per unit depending on size and backlight technology [15]. With 130 displays operating 16 hours daily, fixed-brightness operation at peak intensity results in an estimated annual energy consumption of 95,600 kWh and electricity costs exceeding $11,400 at $0.12/kWh. The absence of ambient light feedback means displays run at unnecessary brightness during low-light periods, wasting an estimated 25–40% of display energy [16].

P2 - Disconnected, unmanaged display nodes: Each display operates without awareness of its surroundings or connection to a central management authority. Content updates require physical access or per-device remote login. There is no mechanism for occupancy-driven content selection, scheduling, or automatic power management.

P3 - Security gap in unauthenticated IoT device communication: Raspberry Pi edge nodes receiving content from a central server typically do so without any authentication mechanism. A malicious actor on the same network could inject content, change playback, or disrupt service without detection [17].

P4 - High total cost of ownership (TCO): Commercial licensing alone exceeds $1,000 per display per year. For a 130-display campus, this represents $130,000 annually in software costs alone, before hardware, energy, installation, and maintenance are considered [4].

## Objective

### General Objective

To design, implement, and validate an open-source Smart Digital Signage System that integrates environmental sensing, centralized content management with role-based access control (RBAC), real-time device command and control, live stream distribution, emergency broadcast with hardware trigger, and a hardened campus network - replacing costly commercial alternatives with a functional, tested prototype.

### Specific Objectives

Table 1-1 Specific objectives and their validation evidence.

| # | Objective | Validation Evidence |
| --- | --- | --- |
| 1 | Develop an embedded sensing layer (motion + ambient light + emergency button) with adaptive display brightness and screen power control | Prototype assembly with real Arduino Mega 2560, three HC-SR04 sensors, LDR module, and potentiometer; serial monitor confirming accurate sensor readings; end-to-end brightness adaptation tested on Debian 13 laptop screens |
| 2 | Build a centralized full-stack CMS with RBAC, group-scoped permissions, and real-time device control | Admin and creator dashboards with role-based routing; 30+ backend integration tests passing; Socket.IO device authentication tests; control lock priority logic tests |
| 3 | Implement live stream distribution (HLS, RTSP, YouTube, RTMP ingest) at zero licensing cost | Stream relay pipeline with FFmpeg; health monitoring; live stream CRUD operations tested via Playwright E2E |
| 4 | Design a dual player architecture (Anthias + MPV) supporting per-device backend selection and shared communication | Device comparison table; content sync verification; dual backend selection in admin dashboard |
| 5 | Deploy a production-ready campus network design (subnet 10.20.0.0/22, dual-NIC Layer 3 isolation) with documented security hardening | Network topology diagram; nftables firewall rules; vulnerability assessment with 12 remediated issues; RBAC permission matrix |
| 6 | Build an emergency broadcast system with hardware trigger, group-wide override, and automatic fallback | Physical emergency button presses trigger immediate local playback on Debian 13 node; group state change propagates to all devices via Socket.IO; disconnection fallback tested |
| 7 | Implement robust concurrency control: control locks, priority synchronization, and deadlock prevention for simultaneous admin/creator access | Control lock acquisition flow diagram; multi-creator priority ordering tests; 423 Locked HTTP response validation |
| 8 | Validate the system through automated testing (backend + frontend E2E) and hardware verification protocols | Jest test suite output (30+ tests); Playwright E2E report (25 tests, 55 screenshots); hardware assembly checklist; serial communication verification |

## Scope and Delimitations

### In scope

We designed a campus-wide deployment architecture capable of supporting 130 nodes across 20 buildings and 10 open areas. We built a functional prototype comprising two edge display nodes (simulated using laptops running Debian 13 live USB sessions) connected to our central server, with a real Arduino Mega 2560 sensor bridge providing motion, ambient light, and emergency button inputs. We validated the full software stack through automated testing on our development machines and verified the sensor-to-brightness control loop end-to-end.

### Out of scope

Full physical deployment across all 20 buildings is beyond the scope of a single capstone project. We did not measure power consumption with a watt meter or multimeter; all power figures were calculated from manufacturer datasheets and duty cycle models. A mobile-responsive admin dashboard and touch-screen kiosk mode were not implemented. Integration with professional broadcast encoders and AI-driven predictive content scheduling are reserved for future work.

### Justification

These delimitations reflect standard final-year project constraints: an eight-month timeline, a prototype hardware budget covering two nodes rather than 130, and a focus on proving the system works and documenting the architecture rather than operating at production scale. Calculated projections are acceptable in academic capstone work; field measurements would require months of continuous operation at scale.

## Significance and Contributions

### Why This Matters

Our Smart Digital Signage System achieves a projected 60% TCO reduction compared to commercial platforms by eliminating licensing fees, reducing energy consumption through adaptive brightness, and using open-source components throughout the stack. For a 130-display campus, this represents approximately $390,000 in savings over five years.

### Novel Contributions

We make the following contributions:

Device token authentication for Pi-class edge nodes: We closed a critical security gap by implementing 64-character hex device tokens generated server-side on first contact, stored in a sidecar file on each node, and validated on every Socket.IO connection. This is the first documented implementation of per-device token authentication in an open-source digital signage platform [17].

Dual player architecture with per-device backend selection: We designed a unified control layer that supports both Anthias (Dockerized web viewer, rich HTML content) and MPV (native media player, fast boot) on a per-device basis, with the backend determining which player to use based on content requirements.

Integrated environmental sensing with real-time brightness control: Our Arduino sensor bridge transmits motion, brightness, and emergency status to the Debian nodes every 500ms, enabling adaptive display brightness based on the Weber-Fechner logarithmic mapping and occupancy-driven content scheduling.

Production-ready network architecture with Layer 3 isolation: We designed a complete campus network with dual-NIC server separation, nftables firewall rules, TLS 1.3 termination, and bandwidth engineering - documented to a level suitable for direct deployment by campus IT staff.

### Who Benefits

Universities, public institutions, hospitals, and any organization running multi-display networks benefit from lower TCO, reduced energy consumption, unified management, and improved emergency preparedness. Our open-source implementation eliminates vendor lock-in and enables customization for domain-specific requirements.

Figure 1-3 High level view of the smart signage system, with network, device and users overview

## Report Structure

The remainder of this report is organized as follows.

Chapter 2 - Literature Review and Related Work surveys existing digital signage platforms, IoT edge computing architectures, power management techniques, live streaming protocols, and role-based access control systems. We identify seven research gaps that our project addresses.

Chapter 3 - Methodology describes our iterative prototyping approach, system requirements, high-level architecture, and development and testing methodology.

Chapter 4 - System Design and Implementation is the core technical chapter. We present the hardware implementation with our real Arduino sensor bridge and Debian 13 simulated edge nodes, the network and infrastructure design, the backend system with authentication and concurrency control, the frontend user interface with 55 Playwright-captured screenshots, the dual player architecture, and the security implementation.

Chapter 5 - Results and Validation report our functional testing results (30+ backend tests, 25 frontend E2E tests), hardware verification, network verification, objectives achievement assessment, and performance and cost analysis.

Chapter 6 - Discussion closes the problem-solution loop, describing how each problem from Section 1.2 is solved, assessing implementation readiness, acknowledging limitations, and comparing our system with related work.

Chapter 7 - Conclusion and Future Work summarize what we built and validated, and outlines immediate, medium-term, and research directions for future development.

Appendices A through P contain detailed technical documentation, network engineering specifications, hardware setup guides, backend and database documentation, frontend user manual, testing protocols, security analysis, software engineering practices, power analysis, cost analysis, literature review references, project summary, master bibliography, source code listings, data sheets, and a glossary of terms.

# Chapter 2

# Literature Review and Related Work

## Digital Signage Platforms

### Open-Source Solutions

We surveyed the open-source digital signage landscape to understand the capabilities and limitations of existing platforms before designing our system. Three platforms emerged as the most relevant: Anthias (formerly Screenly OSE), PiSignage, and Xibo.

- Anthias [1] is a Docker Compose-based digital signage player running on Raspberry Pi. It uses Redis and Celery for background task processing, Flask for the web management interface, and Chromium for on-screen content rendering. Anthias support images, videos, web pages, and overlapping assets in playlists. Its primary strengths are zero licensing cost, a mature codebase with over ten years of development history, and an active open-source community. However, Anthias has critical limitations that directly motivated our project: it offers no sensor integration for brightness adaptation or motion detection, no role-based access control (only a single admin login), no device authentication (any Raspberry Pi on the network can receive content), no campus-scale network design documentation, and no live streaming support.

- PiSignage [2] uses a Node.js server with MongoDB and Pi clients with offline caching. It supports images, videos, multi-zone layouts, and web pages, managed through a web dashboard with a playlist editor and basic device grouping. PiSignage offers offline playback and a REST API, but it is freemium - advanced features cost $10 or more per screen monthly. More critically, PiSignage ships with default credentials (pi:pi), which poses a significant security risk. Like Anthias, it has no sensor integration, no role-based access, no device token authentication, and limited network documentation.

- Xibo [3] is a PHP/MySQL-based CMS with .NET, Android, and Chrome clients. It offers mature content management, fine-grained scheduling, and a visual layout designer. However, Xibo requires a Windows client for some features, has no sensor integration, no brightness control, and imposes per-screen licensing fees.

- Beyond these established platforms, we reviewed research prototypes that attempt to add intelligence to digital signage:

- Zhang et al. (2021) [4] used a passive infrared (PIR) motion sensor to trigger content switching in a retail environment. While innovative, their approach lacks brightness adaptation, centralized management, and was validated only at lab scale.

- Ojha et al. (2022) [5] built a weather-aware content selection system using the OpenWeatherMap API. Their system influences playlist selection based on external weather data but has no local environmental sensing, no power management, and is cloud-dependent.

- Kim and Lee (2020) [6] used Bluetooth Low Energy (BLE) beacons for proximity-based personalized content delivery to smartphones interacting with signage. Their approach requires user opt-in, offers no display power control, and raises privacy concerns.

### Commercial Enterprise Systems

We also evaluated four commercial platforms to understand the cost and capability landscape our system must compete with:

- Yodeck [7] charges $7.99 per screen per month for basic features and $13.99 for professional features. For 130 screens, this translates to $21,824 annually. Yodeck offers drag-and-drop layout editing, scheduling, offline sync, and API access. However, it has no sensor integration, offers no on-premises deployment (cloud-only), and lacks role-based group scoping.

- NoviSign [8] targets the retail and hospitality verticals with pricing between $20.00 and $41.00 per screen per month. At $30 per screen, a 130-display campus would pay $46,800 annually. NoviSign provides touch interactivity, social media widgets, and an analytics dashboard, but it lacks ambient light sensing, occupancy management, and imposes vendor lock-in.

- Rise Vision [9] focuses on the education market at $10.50 to $13.50 per screen per month, yielding approximately $18,720 annually for 130 screens. It integrates with Google Workspace and offers emergency alert templates, but it has no sensor-driven brightness control, no device authentication, and is cloud-dependent.

- ScreenCloud [10] charges $20.00 per screen per month with no free tier, resulting in $31,200 annually for 130 screens. It provides an app marketplace and social media integrations but lacks power management, on-premises source access, and sensor integration.

Across all commercial platforms, we identified five common limitations: no ambient light sensing or adaptive brightness, no occupancy-based power management, no on-premises full source access (proprietary black-box implementations), no campus-scale network design guidance, and per-screen recurring costs that scale linearly with deployment size. Vendor lock-in further traps content and schedules in proprietary formats, making migration costly.

Table 2-1 Commercial digital signage platform comparison.

| Platform | Monthly Cost/Screen | Annual Cost (130 screens) | 5-Year TCO | Free Tier | Open Source | Sensor Integration | On-Premises | RBAC | Live Stream | Network Design |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yodeck | $13.99 | $21,824 | $109,120 | No | No | No | No | No | No | No |
| NoviSign | $30.00 | $46,800 | $234,000 | No | No | No | No | No | No | No |
| Rise Vision | $12.00 | $18,720 | $93,600 | No | No | No | No | No | No | No |
| ScreenCloud | $20.00 | $31,200 | $156,000 | No | No | No | No | No | No | No |
| Our System | $0.00 | $0 | $249,000 (hardware + energy) | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

### Research Gaps Identified

Our survey of open-source platforms, commercial systems, and research prototypes revealed seven critical capabilities that no existing platform combines:

- Zero licensing cost through fully open-source implementation.

- Environmental sensor integration for brightness, motion, and weather input.

- Role-based access control with group scoping for multi-department campuses.

- Device token authentication to secure IoT edge nodes.

- Campus-scale network security design with documented hardening.

- Dual player architecture supporting both browser-based and native video playback.

- Integrated live streaming with RTMP-to-HLS relay at zero cost.

Table 2.1 quantifies these gaps across the evaluated platforms. Our system is the only one that satisfies all seven requirements.

## IoT and Edge Computing for Building Automation

### Raspberry Pi as Edge Node

The Raspberry Pi 4B [11] is built around the Broadcom BCM2711 system-on-chip with a quad-core ARM Cortex-A72 CPU at 1.5 GHz, a VideoCore VI GPU capable of H.264 hardware decodes up to 1080p60, 4 GB of LPDDR4 RAM, Gigabit Ethernet, dual-band Wi-Fi, and Bluetooth 5.0. These specifications make it an attractive edge node for digital signage: sufficient compute for content decoding, network connectivity for real-time communication, and GPIO pins for sensor interfacing.

Our literature review of 47 research papers using Raspberry Pi in building automation revealed three dominant usage patterns:

Sensor gateway: The Pi collects environmental data from sensors and forwards it to a cloud service [12, 13]. These deployments do not drive displays.

- Media player: The Pi runs Kodi, VLC, or Chromium for video playback [14, 15]. These deployments have no sensor input.

- Combined sensing and playback: Only 2 of the 47 surveyed papers use the Pi for both sensing and display output simultaneously [16].

This distribution reveals a significant gap: the Pi’s full potential as a unified sensor-plus-display node remains largely unexplored. Our project addresses this gap by designing a unified node architecture where the Pi simultaneously receives sensor data from Arduino, plays content via Anthias or MPV, communicates with the central server via WebSocket, and controls display brightness - all on a single device.

### Arduino-Based Sensor Acquisition

The Arduino Mega 2560 [17] is built around the ATmega2560 microcontroller running at 16 MHz with 256 KB of flash memory, 8 KB of SRAM, and 4 KB of EEPROM. It provides 54 digital I/O pins (15 with PWM), 16 analog inputs, and 4 hardware UARTs. Its deterministic timing - free from operating system scheduling overhead - makes it ideal for real-time sensor polling.

Our review of 31 Arduino projects in the research literature found three dominant usage patterns: interactive input (capacitive touch, gesture recognition, RFID readers) [18, 19], robotics (motor control, encoder feedback, PID loops) [20], and environmental monitoring. Surprisingly, only 3 of the 31 projects use Arduino for continuous environmental sensing [21].

Our design assigns the Arduino to dedicated real-time environmental sampling at 2 Hz while the Pi handles networking and playback. This separation of concerns simplifies firmware development, ensures deterministic sensor timing, and allows the Pi to focus on content management and network communication.

### Serial Communication Protocols

We evaluated four serial protocols for the Pi-Arduino link: USB CDC, UART via GPIO, I2C, and SPI.

Table 2-2 Serial protocol comparison for the Pi-Arduino data link.

| Protocol | Max Speed | Wiring | Hot-Plug | Max Distance | Linux Driver | Python Library | Complexity | Selected |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| USB CDC | 12 Mbps | 1 cable | Yes | 5 m | Native (cdc_acm) | pyserial | Low | Yes |
| UART (GPIO) | 4 Mbps | 3 wires + level shifter | No | 2 m | Native (ttyAMA0) | pyserial | Medium | No |
| I2C | 3.4 Mbps | 2 wires + GND | No | 1 m | Native (i2c-dev) | smbus2 | High | No |
| SPI | 25 Mbps | 4 wires | No | 0.5 m | Native (spidev) | spidev | High | No |

We selected USB CDC for five reasons. First, it is plug-and-play: no manual pin configuration or level shifting is required (the Pi GPIO operates at 3.3 V while the Arduino uses 5 V logic). Second, Linux provides a native driver, and the Arduino appears automatically as /dev/ttyUSB0 or /dev/ttyACM0. Third, udev persistent naming rules can map a specific Arduino serial number to /dev/arduino0, ensuring the device path remains stable across reboots. Fourth, the USB cable also powers the Arduino (up to 500 mA). Fifth, the Arduino can be disconnected and reconnected without rebooting the Pi.

## Power Management in Display Systems

### Ambient Light Sensors for Brightness Adaptation

Modern smartphones use photodiodes or dedicated ambient light sensors (ALS) to dynamically adjust OLED and LCD backlight intensity, reducing eye strain and extending battery life [22]. This precedent motivated our investigation of ambient light sensing for digital signage.

- Park et al. (2019) [23] attached an external LDR module to a commercial display and reduced power consumption by 18–25% in an office environment using a simple linear mapping from LDR reading to backlight brightness. However, their linear mapping causes perceptible brightness jumps at low ambient light levels, degrading user experience.

- Li and Chen (2020) [24] implemented a logarithmic lux-to-brightness curve based on the Weber-Fechner law and achieved 30% power savings in lab conditions. They used a photodiode instead of an LDR, which requires an amplification circuit and was tested only as a lab prototype without integration into a content management system.

- Wang et al. (2021) [25] used multi-sensor fusion combining an LDR, a PIR motion sensor, and a microphone for context-aware brightness control. They achieved 35% energy savings but required a three-week calibration period per room, making the approach impractical for campus-wide deployment.

A common limitation across all these studies is that they are standalone prototypes without integration into a content management system. Brightness is the only control variable, with no connection to content scheduling, emergency overrides, or occupancy-driven power management.

### Psychophysical Basis: The Weber-Fechner Law

The human visual system perceives brightness on a logarithmic scale rather than a linear one. Doubling physical luminance does not double perceived brightness. The Just Noticeable Difference (JND) - the smallest perceptible change in brightness - is approximately 1% at high brightness levels and approximately 5% at low brightness levels.

The Weber-Fechner law models this relationship mathematically:

Bdisplay=Bmin+Bmax-Bmin⋅Lambient

where (Bmin= 10%) is the minimum readable brightness, (Bmax = 100%) is the maximum brightness, (Lambient) is the normalized LDR reading (0–1023 mapped to 0–1), and (Lmax = 1.0) is the maximum ambient light calibration point.

The logarithmic mapping provides two advantages over linear mapping. First, it produces smooth perceptual transitions from dark to bright conditions, eliminating jarring brightness jumps. Second, it commands lower brightness than linear mapping at low ambient light levels, yielding additional energy savings in the region where displays spend most of their operating hours.

Figure 2-1 Logarithmic brightness adaptation curve based on the Weber-Fechner law (solid) compared with linear mapping (dashed), showing the energy savings zone at low ambient light levels.

Fig. 2.1 Logarithmic brightness adaptation curve based on the Weber-Fechner law (solid) compared with linear mapping (dashed), showing the energy savings zone at low ambient light levels.

### CEC and DDC/CI Protocols for Display Control

We evaluated two industry-standard protocols for programmatic display control: Consumer Electronics Control (CEC) and Display Data Channel Command Interface (DDC/CI).

CEC [66] operates over the HDMI auxiliary channel (pin 13) and supports commands for power on (0x04), power off (0x36), input switching, and vendor-specific brightness control. The cec-client tool from the libCEC library sends commands such as echo "tx 15:44:6E" | cec-client -s -d 1. CEC is compatible with Samsung QB-series commercial displays, LG displays, and most consumer televisions. Standby commands work universally, but some displays ignore brightness commands.

DDC/CI uses I2C communication over the DDC pins (12 and 15) within the HDMI cable. It implements the VESA Monitor Control Command Set (MCCS), where VCP code 10 controls brightness. The ddcutil tool sets brightness with ddcutil setvcp 10 50. DDC/CI is compatible with professional monitors from Dell, NEC, and Eizo, and with Samsung QB-series displays via an adapter. It provides fine-grained brightness control from 0% to 100% and works even when CEC is disabled. However, it requires i2c-dev group membership, and some displays require DDC/CI to be explicitly enabled in the on-screen display menu.

Our system uses CEC as the primary control mechanism for power on/off and coarse brightness adjustment, with DDC/CI as a fallback for fine-grained brightness control. The boot script automatically detects which protocol the connected display supports and configures the control path accordingly.

## Live Streaming in Digital Signage

### RTMP Ingest and HLS Distribution

Real-Time Messaging Protocol (RTMP) is a TCP-based low-latency ingest protocol originally developed by Adobe. It uses standard port 1935 and is supported by encoders including OBS Studio, Wirecast, and hardware encoders from Teradek and Blackmagic [10]. RTMP’s primary limitation is its declining browser support - Flash-based origins are deprecated, and modern playback requires HTTP Live Streaming (HLS) or Dynamic Adaptive Streaming over HTTP (DASH) transcoding.

HTTP Live Streaming (HLS), developed by Apple, uses HTTP to deliver adaptive bitrate video. An .m3u8 playlist file references .ts transport stream segments, typically 10 seconds each. Multiple variant playlists at different resolutions (480p, 720p, 1080p) enable automatic client-side bitrate switching. HLS works through firewalls, is CDN-friendly, and is natively supported on iOS Safari, Android, and through JavaScript players such as hls.js [10].

FFmpeg is the open-source multimedia framework we use for transcoding. The command ffmpeg -i rtmp://input -c:v libx264 -c:a aac -f hls -hls_time 10 output.m3u8 converts an RTMP stream to HLS segments. FFmpeg supports over 100 input formats and 50 output formats, and can use hardware acceleration through V4L2 on Raspberry Pi for H.264 decode.

### Existing Platform Streaming Capabilities

Our survey of existing platforms revealed limited streaming support. Yodeck offers no native streaming and can only embed YouTube or Vimeo iframes. NoviSign provides a premium streaming add-on at $5 per screen per month. Rise Vision uses a web page widget for external streams with no server-side transcoding. ScreenCloud offers RTMP ingest only on its $41 per month plan. Anthias and PiSignage offer no streaming support at all.

Our approach integrates RTMP-to-HLS relay at zero additional cost, supports four source types (RTMP, HLS, RTSP, and YouTube), and performs server-side transcoding to offload work from the Pi clients. This is a unique capability among open-source digital signage platforms.

## Multi-Tenancy and Role-Based Access Control

### IoT Multi-Tenancy Literature

Aytac and Korcok (2022) [26] published a comprehensive survey of IoT multi-tenancy patterns, identifying four layers of tenant isolation: data, network, compute, and physical. They describe role hierarchies ranging from administrator through manager, operator, and viewer, and recommend OAuth 2.0, JWT tokens, and device certificates for identity management. However, their survey does not apply these patterns to group-scoped digital signage with per-device token approval workflows.

Franqueira and Wieringa (2012) [27] studied RBAC effectiveness and found that over-permissive roles - such as a single administrator with unrestricted access - are the primary cause of insider threats in organizational systems. They recommend that role definitions should map to organizational functions rather than individual users, and that the principle of least privilege should guide permission assignment.

Our system implements three roles (administrator, creator, and viewer) with group-level scoping. Creators can publish content only to groups they are explicitly assigned to. Device tokens are separate from user JWTs, and fine-grained toggles such as auto_approve and cross_group_management enable administrators to enforce least privilege without hindering legitimate workflows.

### Campus Network Security for IoT

Stango et al. (2021) [28] demonstrated IoT network segmentation using VLANs and firewalls in a university campus deployment. They found that isolating IoT devices on dedicated subnets reduced the attack surface by 73% compared to flat network architectures. Their work informed our decision to design a dual-NIC server with Layer 3 isolation between the campus-facing network and the signage-facing network.

## Research Gaps Summary

Table 2-3 Research gaps and corresponding project objectives.

| # | Research Gap | Objective Addressed | Section |
| --- | --- | --- | --- |
| 1 | No open-source platform combines sensing, CMS, and RBAC | Objective 2 (full-stack CMS with RBAC) | 4.3 |
| 2 | No platform integrates ambient light sensing with content management | Objective 1 (embedded sensing with adaptive brightness) | 4.1 |
| 3 | No documented device token authentication for Pi-class nodes | Objective 2 (RBAC and device auth) | 4.3.3 |
| 4 | No campus-scale network security design for digital signage IoT | Objective 5 (production-ready network) | 4.2 |
| 5 | No open-source digital signage supports live streaming at zero cost | Objective 3 (live stream distribution) | 4.3.9 |
| 6 | No emergency broadcast with hardware trigger and group override | Objective 6 (emergency broadcast system) | 4.3.10 |
| 7 | No concurrency control for simultaneous multi-user device access | Objective 7 (concurrency control) | 4.3.6 |

These gaps, combined with the cost analysis in Table 2.1, establish the clear need for a new system architecture that unifies environmental sensing, intelligent display control, secure content management, and live stream distribution in a single open-source platform. The following chapters describe how we designed and implemented such a system.

# Chapter 3

# Methodology

## Research Approach

We adopted an iterative prototyping methodology for this capstone project, organized into four two-month phases over an eight-month development cycle. This approach allowed us to build, test, and refine each subsystem incrementally while maintaining a working prototype at every stage.

- Phase 1 - Requirements and Architecture (Months 1–2): We conducted a comprehensive survey of existing digital signage platforms, identified seven critical research gaps documented in Chapter 2, and established the system requirements. We produced the high-level architecture diagram, selected the technology stack, and designed the database schema.

- Phase 2 - Core Backend and Frontend (Months 3–4): We implemented the Node.js backend with Express.js routing, Prisma ORM models, JWT authentication, and the Socket.IO real-time bus. In parallel, we built the React frontend with Vite, Zustand state management, and role-based routing. We established the Jest testing framework for the backend and the Playwright E2E framework for the frontend.

- Phase 3 - Hardware Integration and Sensor Layer (Months 5–6): We assembled the Arduino Mega 2560 sensor bridge with three HC-SR04 ultrasonic sensors, an LDR module, a potentiometer, and an emergency push button. We flashed the Arduino firmware, established USB serial communication at 9600 baud to our Debian 13 laptops (simulating Raspberry Pi edge nodes), and implemented the brightness adaptation algorithm using the operating system’s display brightness API. We tested the emergency button trigger and verified end-to-end sensor-to-screen data flow.

- Phase 4 - System Integration, Testing, and Documentation (Months 7–8): We integrated all subsystems - sensor layer, backend, frontend, and simulated edge nodes - into a unified prototype. We ran the full automated test suite (30+ backend tests, 25 frontend E2E tests), performed hardware verification, documented the network architecture, and produced this report.

- Our technology stack selections were driven by three criteria: open-source licensing (zero recurring cost), active community support (sustainable maintenance), and proven production use (reliability). We selected Node.js and React because both have large ecosystems, extensive documentation, and are widely used in production web applications. We selected PostgreSQL for its ACID compliance and relational data integrity. We selected Prisma for its type-safe database client and migration tooling. We selected Socket.IO for its reliable WebSocket abstraction with automatic fallback to HTTP long-polling.

## System Requirements

### Functional Requirements

We derived the functional requirements from the problem statement in Section 1.2 and the research gaps in Table 2.3. Table 3.1 lists the primary functional requirements and their priorities.

Table 3-1 Functional requirements.

| ID | Requirement | Priority | Validation Method |
| --- | --- | --- | --- |
| FR-1 | The system shall sense ambient light, motion, and weather input via Arduino sensors | High | Hardware prototype; serial monitor verification |
| FR-2 | The system shall adapt display brightness based on ambient light using logarithmic mapping | High | Visual observation; brightness API readback |
| FR-3 | The system shall detect motion via three HC-SR04 sensors for occupancy-aware scheduling | High | Serial monitor; motion flag verification |
| FR-4 | The system shall provide role-based access control (admin, creator, viewer) with group scoping | High | Playwright E2E tests; route guard verification |
| FR-5 | The system shall manage digital content lifecycle (draft, publish, schedule, expire) | High | Backend CRUD tests; Playwright form submission tests |
| FR-6 | The system shall support two content designers (visual Fabric.js and textual Markdown + KaTeX) | Medium | Playwright designer workflow tests |
| FR-7 | The system shall distribute live streams (HLS, RTSP, YouTube, RTMP) at zero licensing cost | Medium | FFmpeg relay tests; stream playback verification |
| FR-8 | The system shall trigger emergency broadcast via hardware button and group-wide override | High | Physical button press test; Socket.IO event verification |
| FR-9 | The system shall prevent concurrent device control conflicts via control locks | Medium | Backend integration tests; priority ordering tests |
| FR-10 | The system shall support offline playback with 72-hour content cache | Medium | Simulated server disconnection test; cache verification |

### Non-Functional Requirements

Table 3-2 Non-functional requirements.

| ID | Requirement | Target | Validation Method |
| --- | --- | --- | --- |
| NFR-1 | Sensor data latency (Arduino to Debian node) | < 1 s | Serial monitor timestamp analysis |
| NFR-2 | Device command latency (server to node) | < 2 s | Socket.IO ack timing logs |
| NFR-3 | Emergency broadcast latency (button to screen) | < 3 s | Stopwatch measurement |
| NFR-4 | Backend API response time (p95) | < 200 ms | Jest performance assertions |
| NFR-5 | Frontend initial page load time | < 3 s | Lighthouse performance audit |
| NFR-6 | Offline resilience duration | 72 h | Simulated disconnection test |
| NFR-7 | Supported concurrent devices (design target) | 130 | Architecture validation; load testing |
| NFR-8 | Code test coverage (backend) | > 70% | Jest coverage report |
| NFR-9 | Security vulnerability severity | Zero P0 (critical) | Security audit; penetration testing |
| NFR-10 | Data persistence reliability | ACID compliance | Prisma transaction tests; PostgreSQL WAL verification |

## High-Level Architecture

Our system follows a centralized client-server architecture with four principal tiers: environmental sensing, edge processing, backend services, and frontend presentation. Figure 3.1 illustrates the complete system topology.

Figure 3-1High-level system block diagram showing the end-to-end data flow from sensors through Arduino, Debian edge node, Socket.IO, Node.js backend, PostgreSQL database, and React frontend.

Figure 3-2 Complete system topology showing 130+ Debian edge nodes, central Ubuntu server with dual-NIC Layer 3 isolation, core and edge switches, and campus subnet 10.20.0.0/22.

### End-to-End Data Flow

The data flow through our system follows a deterministic path from physical sensing to screen rendering:

- Sensing: The Arduino Mega 2560 reads three HC-SR04 distance sensors, one LDR module, one potentiometer, and one emergency push button every 500 ms in a deterministic loop.

- Transmission: The Arduino formats the readings as a comma-separated text packet and transmits it over USB CDC serial at 9600 baud to the Debian 13 laptop simulating the Raspberry Pi edge node.

- Edge Processing: The Debian node parses the serial packet, applies the Weber-Fechner logarithmic brightness mapping, and adjusts the laptop screen brightness via the operating system’s display brightness API. It also evaluates the motion flag and emergency state.

- Server Communication: The Debian node connects to the central Node.js backend via Socket.IO, authenticating with a 64-character hex device token. It emits heartbeats every 10 seconds and receives commands (publish, hide, next, emergency mode) from the server.

- Content Synchronization: The Debian node polls the server every 60 seconds for new signage deployments, downloading only changed assets. For our Anthias-simulated node, content is managed locally; for the MPV-simulated node, files are cached in ~/signage_media/.

- Backend Processing: The Node.js backend handles HTTP REST API requests from the React frontend, manages the PostgreSQL database through Prisma ORM, processes uploaded media with Sharp and FFmpeg, relays live streams, and coordinates emergency broadcasts via Socket.IO room broadcasting.

- Frontend Presentation: The React frontend renders role-appropriate dashboards. Administrators manage devices, groups, and users. Creators design content and publish signage. Viewers browse the public feed and interact with the AI Q&A widget.

- Display Rendering: The Debian node renders scheduled content on its screen via Anthias (web-based, supporting HTML overlays) or MPV (native, fast boot, supporting hardware decode).

### Component Count and Scale

Our campus-scale design targets 130 nodes distributed across 20 buildings and 10 open areas, served by a single Ubuntu server with dual-NIC Layer 3 isolation. Table 3.3 summarizes the component inventory.

Table 3-3 Campus-wide component inventory.

| Component | Quantity per Node | Campus Total | Notes |
| --- | --- | --- | --- |
| Debian edge node (simulated Pi) | 1 | 130 | Laptops or Raspberry Pi 4B in production |
| Arduino Mega 2560 | 1 | 130 | One per display node |
| HC-SR04 ultrasonic sensor | 3 | 390 | Motion/proximity detection |
| LDR module | 1 | 130 | Ambient light sensing |
| Potentiometer | 1 | 130 | Simulated weather input |
| Emergency push button | 1 | 130 | Hardware emergency trigger |
| Commercial display (32”–55”) | 1 | 130 | Samsung QB-series or equivalent |
| Core network switch | - | 2 | Redundant core layer |
| Edge network switch | - | 10 | One per building cluster |
| Central server | - | 1 | Dual-NIC Ubuntu 22.04 LTS |

Our prototype validated the architecture at reduced scale: two Debian 13 laptops (one running Anthias simulation, one running MPV simulation), one Arduino Mega 2560 sensor bridge, one central development server (a laptop running Node.js, PostgreSQL, and nginx), and the laptop screens serving as the display outputs.

## Development and Testing Methodology

### Iterative Development Process

We followed a two-week sprint cycle with the following rhythm:

- Week 1: Feature development. Each sprint focused on one subsystem (e.g., authentication, media processing, sensor integration).

- Weekend: Internal code review. All three group members reviewed pull requests, focusing on correctness, test coverage, and documentation.

- Week 2: Testing and refinement. We wrote unit and integration tests for new features, fixed bugs discovered during review, and updated the sprint backlog.

We maintained the project in a single Git monorepo with Conventional Commits for structured commit messages (feat:, fix:, docs:, test:, refactor:). This practice enabled us to generate changelogs automatically and trace every code change to its functional purpose.

### Testing Strategy

Our testing strategy employed three complementary approaches:

- Backend Integration Testing: We used Jest with Supertest against an isolated PostgreSQL test database (signage_test). Each test runs against a real Express server instance with database transactions rolled back after each test to ensure isolation. We wrote 30+ tests covering authentication, CRUD operations, media upload, deployment scheduling, Socket.IO device lifecycle, emergency state changes, and control lock priority logic.

- Frontend End-to-End Testing: We used Playwright with real browser automation in Chromium. Our test suite comprises 25 tests across two modes: API-only (request mode) for fast direct backend validation on port 5001, and full browser UI (page mode) for end-to-end form submission, navigation, and DOM verification. The tests produced 55 screenshots at 1920 × 1080 resolution, documenting every major UI feature.

- Hardware Verification: We used a structured checklist covering all nine assembly steps, Arduino pin continuity verification, serial communication confirmation at 9600 baud, and end-to-end brightness adaptation observation. We did not use a multimeter or watt meter; all power specifications were derived from component datasheets.

### Development Tooling

We standardized our development environment across all three group members using the following toolchain:

- Version control: Git monorepo with Conventional Commits

- Code quality: ESLint for JavaScript/TypeScript linting, Prettier for code formatting

- Database: Prisma schema migrations for version-controlled database evolution

- Testing: Jest (backend), Playwright (frontend), with coverage thresholds

- Documentation: Markdown in Documents/specializedAnalysis/ folders, one-way dependency from detailed descriptions to the final outline

- Build: Vite for frontend bundling, native Node.js for backend execution

Figure 3-3 Terminal capture of the Git log showing the last 20 commits with Conventional Commit prefixes (feat, fix, docs, test, refactor), demonstrating our structured commit history over the development period.

Fig. 3.2 Terminal capture of the Git log showing the last 20 commits with Conventional Commit prefixes (feat, fix, docs, test, refactor), demonstrating our structured commit history over the development period.

# Chapter 4

# System Design and Implementation

This chapter presents the complete design and implementation of the Smart Digital Signage System. We describe the hardware layer with our real Arduino sensor bridge, the network and infrastructure design, the backend system with authentication and concurrency control, the frontend user interface, the dual player architecture, and the security implementation. This chapter demonstrates that our system is not merely a design but a working prototype with validated hardware and software components.

## Hardware Implementation

### Component Selection and Specifications

We selected the components for our prototype based on cost, availability, and capability. Table 4.1 lists the specifications and pin assignments for each component.

Table 4-1 Component specifications and Arduino pin assignments.

| Component | Specification | Arduino Pin | Purpose |
| --- | --- | --- | --- |
| Arduino Mega 2560 | ATmega2560 @ 16 MHz, 256 KB flash, 8 KB SRAM, 54 digital I/O | - | Sensor acquisition and preprocessing |
| HC-SR04 (×3) | 2–400 cm range, 40 kHz burst, 5 V, <2 mA quiescent | D22–D27 (TRIG/ECHO per sensor) | Motion and proximity detection |
| LDR module | Photoresistor + 10 kΩ divider, 0–1023 analog output | A0 | Ambient light sensing |
| Potentiometer | 10 kΩ linear | A1 | Simulated weather/rain input |
| Emergency button | Push button, 10 kΩ pull-down | D2 | Hardware emergency trigger |
| Debian laptop | Intel/AMD x86_64, 4 GB+ RAM, Debian 13 live USB | USB-B (to Arduino) | Simulated Raspberry Pi edge node |
| Laptop screen | 15.6” LCD, 1920×1080 | HDMI (internal) | Digital signage display |

We selected the Arduino Mega 2560 rather than the Uno because the Mega provides 54 digital I/O pins and 16 analog inputs, giving us sufficient headroom for three HC-SR04 sensors (each requiring one TRIG and one ECHO pin) plus the LDR, potentiometer, and emergency button. The Mega’s four hardware UARTs also provide flexibility for future expansion.

We used laptops running Debian 13 live USB sessions to simulate Raspberry Pi 4B edge nodes. Debian 13 provides the same systemd, Python 3, and network stack that would be available on a Raspberry Pi OS installation, allowing us to validate all software components before physical Pi deployment. The laptop screens served as the digital signage displays.

### Physical Assembly and Wiring

We assembled the sensor bridge on a solderless breadboard following a nine-step procedure:

- Mount sensors: We placed the three HC-SR04 sensors on the breadboard with their ultrasonic transducers facing outward. Sensor 1 (left) and Sensor 3 (right) were angled 45° outward to maximize horizontal coverage. Sensor 2 (center) was mounted perpendicular to the display plane.

- Connect power rails: We ran 5 V and ground rails along the breadboard edges, using 22 AWG jumper wires. The Arduino’s 5 V pin can supply up to 400 mA, which is sufficient for all sensors combined (~50 mA peak).

- Wire HC-SR04 sensors: Each sensor received VCC to the 5 V rail, GND to the ground rail, TRIG to a digital output pin (D22, D24, D26), and ECHO to a digital input pin (D23, D25, D27). We added 1 kΩ series resistors on the ECHO lines as a precaution against voltage mismatch, though the Arduino Mega 2560’s 5 V logic is fully compatible.

- Connect LDR module: We connected the LDR module’s VCC to 5 V, GND to ground, and the analog output to pin A0.

- Connect potentiometer: We wired the potentiometer’s outer terminals to 5 V and ground, and the wiper to pin A1.

- Connect emergency button: We wired the push button between pin D2 and 5 V, with a 10 kΩ pull-down resistor to ground on D2. This configuration ensures the pin reads LOW when the button is open and HIGH when pressed.

- USB connection: We connected the Arduino to the Debian laptop via a USB A-to-B cable. The cable provides both serial communication and power (up to 500 mA).

- Verification: We performed a continuity check between 5 V and GND to confirm no short circuits. We verified sensor orientation (transducers facing outward) and checked that the HDMI output on the laptop was active.

Figure 4-1 Photograph of our assembled prototype showing the Arduino Mega 2560 on a solderless breadboard with three HC-SR04 sensors, connected via USB to a Debian 13 laptop.

Figure 4-2 Sensor wiring schematic showing Arduino Mega pin assignments for HC-SR04 (D22–D27), LDR (A0), potentiometer (A1), and emergency button (D2).

### Sensor Fusion and Preprocessing

We wrote the Arduino firmware (sensors.ino) to run a deterministic 2 Hz main loop. Each iteration reads all sensors, computes derived values, and transmits a formatted packet over USB CDC serial at 9600 baud.

- HC-SR04 distance measurement: For each sensor, the firmware drives the TRIG pin HIGH for 10 µs to initiate an 8-pulse 40 kHz ultrasonic burst. It then measures the ECHO pulse duration using pulseIn() with a 30 ms timeout. The distance in centimeters is computed as d = (t_echo × 0.0343) / 2, where the speed of sound is 343 m/s at 20°C.

- Motion detection: The firmware sets the motion flag to 1 if any of the three sensors reports a distance less than 100 cm. This threshold corresponds to a typical hallway width of 200 cm, detecting a person at mid-distance. To prevent rapid toggling, we implemented debouncing: three consecutive positive readings are required to trigger the motion state (1.5 seconds at 2 Hz), and ten consecutive negative readings are required to clear it (5 seconds).

- Brightness normalization: The LDR module outputs an analog value from 0 to 1023 (10-bit ADC). We map this to a normalized range [0, 1] for transmission to the Debian node, where the Weber-Fechner logarithmic mapping computes the target screen brightness.

- Emergency button: The firmware reads pin D2 every iteration. When the button is pressed (pin reads HIGH), it immediately sets the emergency flag to 1 in the outgoing packet. This gives the Debian node a maximum detection latency of 500 ms (one loop iteration).

- Serial packet format: Each transmission uses the format SENSOR,motion,brightness,rain,emergency\n, where motion is 0 or 1, brightness is the normalized LDR value (0–100), rain is the normalized potentiometer value (0–100), and emergency is 0 or 1.

Figure 4-3 Photograph of the sensor array mounted near the laptop screen, showing the three HC-SR04 sensors positioned for left, center, and right coverage.

Figure 4-4 Screenshot of the Arduino IDE Serial Monitor at 9600 baud, showing five consecutive SENSOR: packets with motion, brightness, rain, and emergency values.

### Adaptive Brightness Algorithm

We implemented the Weber-Fechner logarithmic brightness mapping on the Debian node using the formula:

Bdisplay=Bmin+Bmax-Bmin⋅Lambient

where (Bmin= 10%) is the minimum readable brightness, (Bmax = 100%) is the maximum brightness, (Lambient) is the normalized LDR reading (0–1023 mapped to 0–1), and (Lmax = 1.0) is the maximum ambient light calibration point.

When the Debian node receives a sensor packet, it extracts the brightness value and applies the logarithmic mapping. The resulting percentage is passed to the operating system’s display brightness API. On Debian 13, we used the brightnessctl utility (brightnessctl set ${percentage}%) to adjust the laptop screen backlight. We verified the end-to-end control loop by covering the LDR with a finger (brightness decreased within 1 second) and shining a flashlight on it (brightness increased within 1 second).

### Simulated Edge Node Software

We implemented three Python services that run on each Debian 13 laptop, simulating the software stack that would execute on a Raspberry Pi 4B in production:

- socket_client.py: This service maintains a persistent Socket.IO connection to the central Node.js backend. It authenticates using a 64-character hex device token stored in ~/.device_token. It emits heartbeats every 10 seconds (heartbeat event with device status, uptime, and IP address), receives commands from the server (signage_command, emergency_mode_start, emergency_mode_end, refresh_display), and forwards sensor data from the Arduino to the server.

- content_sync.py: This service polls the backend every 60 seconds for new signage deployments. It downloads only changed assets (incremental sync) and caches them locally. For the Anthias-simulated node, it prepares assets in the format expected by the Anthias REST API. For the MPV-simulated node, it downloads files to ~/signage_media/ and maintains a local playlist.

- scheduler.py (MPV only): This service manages the content playback schedule. It reads the active deployment list from the cache, respects start_date and end_date windows, and advances to the next item at the configured display duration. When the server emits refresh_display, it immediately re-reads the deployment list.

### Systemd Service Configuration

We configured all three services to start automatically on Debian boot using systemd unit files. Each unit specifies:

After=network.target: Service starts only after network is available

Restart=on-failure: Automatic restart if the service crashes

RestartSec=10: 10-second delay before restart

StandardOutput=journal: Logs captured in systemd journal

User=signage: Service runs under a dedicated non-root user

We verified the systemd configuration by running systemctl status socket-signage.service and confirming the active (running) state.

Figure 4-5 Screenshot of the systemd status output showing socket-signage.service as active (running), confirming automatic startup on boot.

## Network and Infrastructure Design

### Subnet Design

We designed the campus network around the subnet 10.20.0.0/22, which provides 1,022 usable IP addresses (4,094 total minus network, broadcast, and gateway addresses). This represents 686% headroom above the 130-node target, allowing for future expansion without renumbering.

Table 4-2 Structured IP allocation within 10.20.0.0/22.

| Range | Purpose | Count |
| --- | --- | --- |
| 10.20.0.1 – 10.20.0.19 | Infrastructure (gateway, DNS, NTP, management) | 19 |
| 10.20.0.20 – 10.20.0.49 | Server services (backend, database, nginx) | 30 |
| 10.20.0.50 – 10.20.3.199 | Edge nodes (displays, sensors) | 150 |
| 10.20.3.200 – 10.20.3.254 | Spare / future expansion | 55 |

### Layer 3 Isolation with Dual-NIC Server

Our server design uses two network interface cards (NICs) to enforce Layer 3 isolation between the campus network and the signage network. NIC-1 faces the campus (WAN) and accepts only SSH (port 22), HTTP (port 80), and HTTPS (port 443) traffic. NIC-2 faces the signage subnet (LAN) and accepts API traffic (port 3000), RTMP ingest (port 1935), HLS segment serving (port 443), DHCP (port 67/68), and DNS (port 53).

No direct routing exists between the campus subnet and the signage subnet. All cross-subnet traffic must traverse the server, which applies firewall rules, TLS termination, and rate limiting. This design prevents a compromised campus workstation from directly attacking signage nodes.

Figure 4-6 Network topology diagram showing the dual-NIC Ubuntu server, core switches, edge switches, and Debian nodes in the 10.20.0.0/22 signage subnet.

### Core Services

We deployed five core services on the Ubuntu server:

dnsmasq: Provides DHCP address assignment and local DNS resolution for the signage subnet. This ensures nodes can resolve the server’s address even if campus DNS fails.

chrony: Network Time Protocol (NTP) service ensures all nodes maintain synchronized time, which is critical for content scheduling and log correlation.

nginx: Reverse proxy and static file server. It terminates TLS 1.3 connections on the campus-facing NIC and proxies API requests to the Node.js backend. It also caches HLS segments to reduce repeated backend load.

nftables: Firewall enforcing the rule sets described in Section 4.6.3.

tc (traffic control): Rate-limits content sync traffic to prevent edge switch saturation during bulk deployments.

### Bandwidth Engineering

Table 4-3 Bandwidth analysis by operational scenario.

| Scenario | Concurrent Nodes | Per-Node Rate | Aggregate | Notes |
| --- | --- | --- | --- | --- |
| Normal operation | 130 | 5 kbps (heartbeat) | 650 kbps | Negligible; WebSocket keepalive |
| Content push (images) | 130 | 500 KB every 60 s | ~10.8 Mbps | Bursty; tc limits to 5 Mbps sustained |
| Content push (videos) | 130 | 5 MB every 300 s | ~17.3 Mbps | Staggered by node ID to prevent thundering herd |
| Live stream (1080p HLS) | 130 | 4 Mbps | 520 Mbps | Served from nginx cache; backend not involved |
| Emergency broadcast | 130 | 200 KB (one-time) | 26 Mbps | Single multicast packet preferred |

### Failure Modes and Resilience

Our network design accommodates three failure modes:

Server failure: Nodes continue playing cached content for 72 hours (the Service Level Agreement, or SLA). After 72 hours, they purge content and display a disconnection image.

Network partition: If a node loses connectivity to the server, it continues its cached schedule indefinitely within the 72-hour window. When connectivity is restored, it re-authenticates and re-syncs.

Power outage: We sized uninterruptible power supply (UPS) units by tier: 300 VA for the server (15 minutes runtime), 150 VA per core switch (10 minutes), and 600 VA per building cluster (5 minutes for graceful shutdown).

### Security Hardening at the Network Layer

We implemented nftables rules on both NICs. The campus-facing NIC (NIC-1) accepts only SSH, HTTP, and HTTPS from the campus subnet, with rate limiting on SSH to prevent brute-force attacks. The signage-facing NIC (NIC-2) accepts only API, RTMP, HLS, DHCP, and DNS traffic from the signage subnet, with explicit drops for all other ports and protocols.

Figure 4-7  Firewall rules diagram showing NIC-1 campus-facing rules (SSH/HTTP/HTTPS/NTP) and NIC-2 signage-facing rules (API/RTMP/HLS/DHCP/DNS).

## Backend System Implementation

### Layered Architecture

Our backend follows a layered architecture: Express.js routes receive HTTP requests, middleware validates authentication and input, services encapsulate business logic, repositories abstract database access, and Prisma ORM translates operations into PostgreSQL queries. This separation ensures that business logic is independent of the transport layer and database schema, making the system testable and maintainable.

Figure 4-8 Backend layered architecture diagram showing the data flow from Routes through Middleware, Services, Repositories, Prisma ORM, to PostgreSQL.

### Database Design

Our PostgreSQL database, managed through Prisma ORM, contains the following key entities:

User: id, email, password_hash, role (admin, creator, viewer), created_at, updated_at

Group: id, name, description, signage_state (NORMAL, BREAKING_NEWS, SECURITY_RISK, EMERGENCY), created_by

Device: id, name, device_type (anthias, mpv), group_id, device_token, last_seen, is_approved

Post: id, title, slug, description, signage_state, start_date, end_date, is_enabled, created_by

SignageDeployment: id, post_id, device_id, priority, is_active

LiveStream: id, name, url, type (hls, rtsp, youtube, rtmp), is_active

Attachment: id, post_id, filename, type (image, video, document), extracted_text

Relationships include: User belongs to many Groups (many-to-many via UserGroup), Group contains many Devices (one-to-many), Post belongs to many Groups (many-to-many via PostGroup), and Device has many SignageDeployments (one-to-many).

### Authentication and Authorization

We implemented a dual authentication system to secure both human users and IoT edge nodes.

User authentication (JWT): When a user logs in, the backend validates the email and password against bcrypt-hashed credentials stored in PostgreSQL. Upon successful validation, it issues a JSON Web Token (JWT) signed with a server-side secret and a configurable expiry (default 24 hours). The frontend stores this token in localStorage and sends it as a Bearer token in the Authorization header for every subsequent API request. The authenticateUser middleware verifies the token on every protected route.

Device authentication (device token): When a Debian node first connects via Socket.IO, it sends its pre-assigned device_id in the connection handshake. If the device is not yet approved, the server generates a cryptographically random 64-character hexadecimal token and sends it to the device via the device_token event. The device stores this token in ~/.device_token. On subsequent connections, the device includes this token in its authentication payload. The server validates the token against the database before accepting any commands or data from the device. Unapproved devices cannot receive content or commands.

Figure 4-9 Authentication flow sequence diagram showing JWT login for users and device token assignment for IoT nodes.

### User and Group Management

We implemented three roles with distinct capabilities, as summarized in Table 4.4.

Table 4-4 Role capabilities matrix.

| Capability | Administrator | Creator | Viewer | Public |
| --- | --- | --- | --- | --- |
| Manage all devices | Yes | No | No | No |
| Manage assigned devices | Yes | Yes (own group) | No | No |
| Create/edit all posts | Yes | No | No | No |
| Create/edit own posts | Yes | Yes | No | No |
| Publish to any group | Yes | No | No | No |
| Publish to assigned groups | Yes | Yes | No | No |
| Trigger emergency mode | Yes | No | No | No |
| View public feed | Yes | Yes | Yes | Yes |
| Use AI Q&A | Yes | Yes | Yes | Yes (rate-limited) |
| Approve pending devices | Yes | No | No | No |
| Manage control locks | Yes | No | No | No |

Group-scoped permissions: Users belong to one or more Groups via the UserGroup join table. Devices belong to exactly one Group. Posts can be published to one or more Groups via the PostGroup join table. When a Creator attempts to publish a post, the backend checks that the Creator is a member of at least one of the target Groups. The cross_group_management toggle on the User record allows Administrators to bypass this check for specific trusted users.

Figure 4-10 RBAC permission matrix diagram showing the mapping of roles (admin, creator, viewer, public) to system capabilities.

### Content Lifecycle Management

A Post in our system has the following lifecycle states: draft → published → scheduled → expired. Creators compose posts in draft state using either the visual designer (Fabric.js) or the textual designer (Markdown with KaTeX math rendering). When a Creator publishes a post, the backend creates SignageDeployment records for each target device. Each deployment record includes start_date, end_date, priority, and is_enabled fields.

The Debian node polls GET /api/signage/device/:id/deployments every 60 seconds. The backend filters deployments to those where the current date falls within start_date and end_date, the post status is published, and is_enabled is true. Results are ordered by priority, ensuring that emergency content always precedes normal content.

When deployments change, the server emits refresh_display to affected devices via Socket.IO. Devices immediately pull the updated playlist rather than waiting for the next 60-second poll interval.

### Device Management and Control Locks

Device lifecycle: Administrators pre-register devices by assigning a device_id in the admin dashboard. When a Debian node first connects, it emits a heartbeat. The server assigns a device token and marks the device as pending_approval. The Administrator approves the device, which triggers token persistence and a refresh_display command.

Control locks: When multiple administrators or creators attempt to control the same device simultaneously, the backend’s control_lock mechanism prevents race conditions. A control lock record contains the device ID, user ID, priority level, action type, and expiration timestamp (30-second TTL by default).

The priority hierarchy is: Emergency system (priority 20) > Administrator (15) > Creator (10) > Viewer (5). A user can acquire a lock only if no higher-priority lock is active on the same device. If an equal or lower-priority lock exists, the request is rejected with HTTP 423 (Locked), and the response indicates who holds the lock and when it expires.

Figure 4-11 Control lock acquisition flow sequence diagram showing Administrator A acquiring a lock on Device 3, and Creator B receiving a 423 Locked response.

Multi-creator priority synchronization: When two creators publish posts to the same device group, the priority field on each SignageDeployment record determines playback ordering. Additionally, the signage state hierarchy EMERGENCY > SECURITY_RISK > BREAKING_NEWS > NORMAL ensures that higher-priority states always take precedence, regardless of the individual post priorities.

Online/offline tracking: The server maintains a last_seen timestamp for each device, updated on every heartbeat. If a device misses three consecutive heartbeats (30 seconds), the server marks it as offline. The admin dashboard displays real-time device status.

### Signage Deployment Engine

The deployment engine is the core scheduling subsystem. When a Creator publishes a post to a group, the backend creates SignageDeployment records for all devices in that group. Each record captures:

post_id - reference to the content

device_id - target device

priority - numeric priority within the group

start_date and end_date - temporal validity window

is_enabled - boolean toggle for temporary suppression

The Debian node polls for deployments every 60 seconds with jitter (±5 seconds) to prevent thundering herd. The response is a JSON array ordered by priority. For Anthias-simulated nodes, the agent uploads assets via the Anthias REST API. For MPV-simulated nodes, the agent downloads files to ~/signage_media/ and updates the local playlist.

### Media Processing Pipeline

All uploaded media passes through a backend processing pipeline before being served to devices.

Images (Sharp): Uploaded PNG or JPEG files are auto-oriented based on EXIF data, optionally cropped using percentage-based coordinates from the frontend (react-easy-crop), resized if they exceed 4096 × 4096 pixels, compressed to WebP format at quality 88, and saved to uploads/images/{uuid}.webp. WebP provides approximately 30% smaller file sizes than JPEG at equivalent quality.

Videos (FFmpeg): Uploaded MOV, AVI, or MP4 files are probed for metadata (duration, resolution, codec, bitrate), optionally trimmed using temporal start/end times from the frontend (VideoTrimSlider), optionally cropped using spatial coordinates, transcoded to H.264 video with AAC audio in an MP4 container with faststart (moov atom at file beginning for progressive download), and saved to uploads/videos/{uuid}.mp4.

Documents (PDF/DOCX/PPTX): Text is extracted using mammoth (for DOCX/PPTX) and pdf-parse-fork (for PDF). The extracted text is stored in the Attachment record and provided as context to the AI Q&A system.

Figure 4-12 Media processing pipeline flowchart showing image processing (Sharp), video processing (FFmpeg), and document text extraction (mammoth/pdf-parse-fork) paths.

Static files are served from /uploads/* and /streams/* via Express static middleware with path traversal protection. All paths are resolved with path.resolve() and checked against the upload root directory.

### Live Stream Relay

Our system supports four stream types, summarized in Table 4.5.

Table 4-5 Stream types and relay architecture.

| Type | Ingest Source | Relay Mechanism | Player Support |
| --- | --- | --- | --- |
| HLS | External .m3u8 URL | Direct proxy + nginx segment caching | Anthias (Chromium), MPV |
| RTSP | IP camera URL | FFmpeg → HLS segments | MPV (native), Anthias (hls.js) |
| YouTube | YouTube HLS URL | Proxy + yt-dlp resolution | Anthias (Chromium) |
| RTMP | OBS/Encoder push to port 1935 | node-media-server → FFmpeg → HLS | Anthias (hls.js), MPV |

The RTMP ingest server listens on port 1935. When a stream is configured, the backend spawns an FFmpeg child process that transcodes the input to HLS segments stored in streams/{id}/. A background health monitor checks each relay process every 30 seconds; if a process has crashed, the monitor kills any zombie processes, restarts the relay, and logs the incident. Auto-restart is capped at five attempts within ten minutes to prevent infinite loops.

Figure 4-13 Stream relay pipeline diagram showing OBS → RTMP → FFmpeg → HLS segments → nginx → Anthias/MPV playback.

### Emergency Broadcast System

Our emergency broadcast system supports three independent trigger sources:

Hardware trigger: The Arduino emergency button sends an emergency:1 flag in the sensor packet. The Debian node detects this flag and immediately plays the local emergency_fallback.mp4 file, even if the server is unreachable. It then attempts to notify the server via the emergency_trigger Socket.IO event.

Server command: An Administrator sets a group’s signage_state to EMERGENCY in the admin dashboard. The backend updates the database and broadcasts emergency_mode_start to all online devices in that group.

Group state change: Any authorized API client can set a group’s state, enabling integration with external safety systems such as fire alarm APIs.

The signage state hierarchy is strict: EMERGENCY > SECURITY_RISK > BREAKING_NEWS > NORMAL. This hierarchy is enforced in both backend deployment queries and frontend state selection UI. Emergency mode does not clear automatically; it requires all assigned groups to return to NORMAL state before devices resume scheduled content.

Figure 4-14 Emergency broadcast state machine diagram showing transitions between Normal, Emergency, Disconnected, SecurityRisk, and BreakingNews states.

### Socket.IO Real-Time Bus

Our Socket.IO implementation supports the following events:

Inbound events (Debian node → Server): - heartbeat - device status, uptime, IP address, sensor readings (every 10 s) - device_online / device_offline - connection state transitions - signage_asset_synced - confirmation that Anthias received uploaded assets - error_log - remote error collection for debugging - emergency_trigger - hardware button pressed

Outbound events (Server → Debian node): - device_token - assign or re-assign authentication token - signage_command - publish, hide, show, delete, next, previous, start - emergency_mode_start / emergency_mode_end - group-wide emergency state - refresh_display / restart_display - content or player reload

Ack support: We implemented emitToDeviceAck, which uses Socket.IO timeouts to confirm that the Debian node received a command. This is essential for signage control, where “fire and forget” command delivery is unacceptable.

### Fault Tolerance and Race Condition Handling

We designed the backend to handle several concurrent access scenarios:

Two administrators sending next to the same device → the control lock prevents the second command from executing until the first lock expires.

Creator A publishing a high-priority post while Creator B publishes a low-priority post → priority ordering resolves the conflict; the higher-priority post plays first.

An administrator triggering emergency mode while a creator publishes normal content → the state hierarchy ensures emergency content takes precedence.

Server restart during active streams → the health monitor detects missing relay processes and auto-restarts them within 30 seconds.

Network partition → Debian nodes continue playing cached content for 72 hours, then display the disconnection fallback image.

Critical database operations use ACID transactions through PostgreSQL and Prisma, including emergency state changes, device token generation, and deployment creation.

### AI-Assisted Public Engagement

Public visitors browse /feed and /post/:id without authentication. The AI Q&A widget at the bottom of each post detail page sends the user’s question, conversation history, and extracted document text from attachments to POST /api/ai/ask. The backend forwards this context to the OpenAI GPT API and streams the response back to the frontend in real time. Rate limiting (10 questions per IP per hour) prevents abuse.

## Frontend and User Interface

We built the frontend in React 19 with Vite for fast development builds and hot module replacement. State management uses Zustand for its minimal API and lack of boilerplate. HTTP requests use Axios with request/response interceptors for JWT token attachment and 401 unauthorized handling. Real-time updates use Socket.IO Client with lazy connection (only connects on pages that need live data).

### Admin Dashboard

The admin dashboard provides a unified interface for system management.

Authentication: The login page accepts email and password, validates credentials against the backend, and stores the JWT and user profile in localStorage. On application boot, the frontend validates token expiry and redirects to the login page if expired.

Figure 4-15 Login page with empty username and password fields, campus branding visible; Sign In button disabled until both fields are filled.

Group Management: Administrators view all groups in a table with inline editing for name, description, and signage state. New groups are created via a modal form.

Figure 4-16 Admin Groups page listing all campus groups with display state, member count, and assigned devices.

User Management: Administrators view all users with their roles and group assignments. Inline editing supports role changes and group membership updates.

Figure 4-17 Admin Users page showing all registered accounts with roles, groups, and permissions

Device Management: The devices page lists all registered and pending devices. Administrators approve pending devices (assigning device tokens), view device status (online/offline, last seen), and send playback commands (next, previous, start).

Figure 4-18 Admin Devices page showing the registered hardware fleet with online/offline status, register form, and settings inspector.

### Creator Dashboard

The creator dashboard is the primary content authoring interface.

Posts: Creators compose posts with a title, Markdown description, URL slug, and attachments. The form supports image upload with cropping, video upload with trimming, and document upload for AI context.

Figure 4-19 Creator Posts page with the new-post editor on the left and the post list with filters on the right.

Figure 4-20 Image cropping interface with aspect ratio locking to 16:9 for signage compatibility, drag and zoom controls visible.

Figure 4-21 Video trimming slider used to extract specific clips before attaching to a post, start and end handles visible.

Visual Designer (Fabric.js): Creators can design signage slides with draggable text, shapes, and images on a canvas. The designer supports layering, opacity, and export to PNG.

Figure 4-22 Creator Visual Designer with an empty Fabric.js canvas and toolbar on the top for adding text, shapes, and images.

Textual Designer (Markdown + KaTeX): Creators can compose text-heavy slides using Markdown syntax with live KaTeX math rendering. The exported result is a styled HTML slide.

Figure 4-23 Designer in Markdown mode with empty editor on the top and live-preview panel on the buttom, supporting KaTeX math notation.

Signage Publishing: Creators select target groups and set scheduling parameters (start date, end date, priority) before publishing.

Figure 4-24 Signage Publish page listing publishable posts on the left and target devices on the right, with duration and priority controls.

Live Stream Management: Creators can add, edit, and monitor live streams. The interface displays stream health status and provides embed codes.

Figure 4-25 Live Streams page with the list of existing streams showing source type and status, plus the creation form.

### Public Feed and Viewer Interface

Public visitors access /feed to browse all published posts in a responsive grid layout. Clicking a post navigates to /post/:id for the full content view.

Figure 4-26 Public feed showing all campus announcements sorted by priority and recency, no login required.

### AI Q&A Widget

The AI Q&A widget appears at the bottom of each post detail page. Visitors type questions, and the system sends the question, conversation history, and extracted document text to the OpenAI GPT API. Responses stream in real time.

Figure 4-27 AI chat window opened on a post detail page, ready for user questions grounded on post text and attachments.

### Emergency Handling UI

Administrators trigger emergency mode from the Groups page by setting a group’s signage state to EMERGENCY. A confirmation dialog prevents accidental activation.

Figure 4-28 Groups page showing the Emergency Broadcast group in EMERGENCY state with red indicator and inline state controls.

### State Management

We chose Zustand over Redux for state management because its API requires no boilerplate (no action types, reducers, or dispatch functions). Our store modules include:

authStore: JWT token, user profile, login/logout actions

uiStore: Theme, sidebar state, notification queue

dataStore: Cached API responses with automatic invalidation

Axios interceptors attach the Bearer token to every outgoing request and handle 401 responses by redirecting to the login page. Socket.IO Client connects lazily (only on pages requiring real-time data) and reconnects automatically with exponential backoff.

## Dual Player Architecture

Our system supports two playback backends on a per-device basis, enabling administrators to choose the most appropriate player for each display’s content requirements.

Table 4.6 Anthias versus MPV comparison.

| Feature | Anthias | MPV |
| --- | --- | --- |
| Rendering engine | Dockerized Chromium | Native OpenGL/VAAPI |
| Content types | HTML, images, videos, web pages | Images, videos, streams |
| Boot time | 15–30 s (Docker + browser) | 1–2 s |
| Memory footprint | 400–600 MB | 50–150 MB |
| Web content | Full CSS/JS support | Not supported |
| Hardware decode | Limited | Full V4L2/VDPAU |
| Remote commands | Anthias REST API | Direct MPV IPC socket |
| Overlay support | CSS-based | Lua scripting |
| Use case | Rich interactive signage | Fast-boot video playback |

Shared communication layer: Both player types use the same socket_client.py for Socket.IO communication, the same content_sync.py for asset synchronization, and the same authentication mechanism. The backend determines which player a device uses based on the device_type field (set during device registration), and the appropriate control commands are sent accordingly.

## Security Implementation

### Threat Model

We identified four trust boundaries in our system architecture: the Campus LAN (untrusted), the Server DMZ (semi-trusted), the Database Zone (trusted), and the Signage LAN (trusted but vulnerable to physical access). Figure 4.28 illustrates these boundaries and the attack surfaces at each layer.

Figure 4-29 Attack surface diagram showing trust boundaries: Campus LAN → Server DMZ → Database Zone → Signage LAN, with identified attack vectors at each layer.

### Vulnerabilities Addressed

We conducted a security audit of the codebase and identified 12 vulnerabilities, summarized in Table 4.7.

Table 4-6 Vulnerability inventory and remediation status.

| ID | Severity | Component | Description | Remediation |
| --- | --- | --- | --- | --- |
| V-01 | P0 (Critical) | Socket.IO | Unauthenticated device connections accepted any Pi on the network | Device token authentication; 64-char hex tokens |
| V-02 | P0 (Critical) | API | No rate limiting on login endpoint enabled brute-force attacks | Express-rate-limit: 5 attempts per 15 min per IP |
| V-03 | P0 (Critical) | File upload | Path traversal possible via malicious filename | path.resolve() + root directory check |
| V-04 | P0 (Critical) | CORS | Wildcard CORS allowed any origin to access API | Whitelist of allowed origins |
| V-05 | P1 (High) | JWT | Tokens had no expiry; stolen tokens valid forever | 24-hour expiry + refresh token rotation |
| V-06 | P1 (High) | Database | Prisma queries lacked parameterized input in one route | All queries use Prisma query builder |
| V-07 | P1 (High) | Socket.IO | No origin validation on WebSocket handshake | io.engine.use(cors(...)) with whitelist |
| V-08 | P1 (High) | Static files | Directory listing enabled on /uploads/ | dotfiles: 'deny' in Express static config |
| V-09 | P1 (High) | Logging | Error logs contained stack traces with file paths | Sanitized logs; no paths in production |
| V-10 | P2 (Medium) | Frontend | localStorage stored JWT without encryption | Acceptable risk; HTTPS mitigates transmission |
| V-11 | P2 (Medium) | API | No Content Security Policy headers | Added helmet middleware with CSP |
| V-12 | P2 (Medium) | Network | nftables rules not persisted across reboots | nftables-persistent package + systemd service |

Figure 4-30 Screenshot of the Socket.IO authentication code showing device token verification in the connection handshake handler.

### Network Hardening

We implemented nftables rules on both server NICs. The campus-facing NIC accepts only SSH (rate-limited), HTTP, and HTTPS. The signage-facing NIC accepts only API, RTMP, HLS, DHCP, and DNS traffic. All other ports and protocols are explicitly dropped. TLS 1.3 termination on nginx ensures encrypted communication between the frontend and backend.

### Remediation Summary

All four P0 (critical) and five P1 (high) vulnerabilities were remediated before system validation. The three P2 (medium) vulnerabilities were documented as accepted risks with mitigating controls. No P0 or P1 vulnerabilities remain in the production codebase.

This chapter has presented the complete design and implementation of the Smart Digital Signage System, from the physical Arduino sensor bridge through the Debian-simulated edge nodes, network infrastructure, backend services, frontend interfaces, dual player architecture, and security hardening. The following chapter describes how we validated this implementation through automated testing and hardware verification.

# Chapter 5

# Results and Validation

This chapter presents the validation of our Smart Digital Signage System through automated software testing, hardware verification, network testing, objectives achievement assessment, and performance and cost analysis.

## Functional Testing

### Backend Integration Testing

We wrote 30+ backend integration tests using Jest with Supertest. Each test runs against a real Express server instance connected to an isolated PostgreSQL test database (signage_test). After each test, database transactions are rolled back to ensure isolation.

The test suite covers:

Authentication: Login with valid and invalid credentials, JWT token generation and validation, token expiry handling, logout and session termination.

CRUD operations: Create, read, update, and delete for users, groups, devices, posts, deployments, and live streams.

Media upload: Image upload with Sharp processing, video upload with FFmpeg transcoding, document upload with text extraction, path traversal attack prevention.

Deployment scheduling: Date window filtering, priority ordering, state hierarchy enforcement (EMERGENCY > SECURITY_RISK > BREAKING_NEWS > NORMAL).

Socket.IO device lifecycle: Device registration, token assignment, heartbeat processing, command delivery with acknowledgment, offline detection.

Emergency state changes: Group state transition to EMERGENCY, broadcast to all devices in the group, automatic fallback when state returns to NORMAL.

Control lock logic: Lock acquisition with priority, lock rejection for lower-priority users, lock expiration, concurrent access scenarios.

RBAC enforcement: Role-based route access denial, group-scoped permission validation, cross-group management toggle.

All 30+ backend tests passed, with code coverage exceeding 70% for business logic files. The test suite runs in approximately 45 seconds on a standard development machine.

Figure 5-1 Screenshot of the Jest test output showing all backend integration tests passing with green checkmarks and the final coverage summary.

### Frontend End-to-End Testing

We wrote 25 frontend end-to-end tests using Playwright with real browser automation in Chromium. The tests run in two modes:

API-only mode (request): Uses Playwright’s direct HTTP client to hit the backend test server on port 5001. This mode is fast (approximately 2 seconds per test) and validates API contracts without browser overhead.

Browser UI mode (page): Uses a full Chromium browser instance to navigate the frontend, fill forms, click buttons, and verify DOM state. This mode validates the complete user journey from login through content creation to publication.

The test suite covers:

Authentication flow: Login page rendering, form submission, JWT storage, token expiry redirect, logout.

Admin dashboard: Group creation, user management, device approval, playback commands.

Creator dashboard: Post creation with image upload and cropping, video upload and trimming, Fabric.js designer workflow, Markdown designer workflow, signage publishing with group selection.

Public feed: Feed grid rendering, post detail navigation, AI Q&A widget interaction.

Emergency handling: Group state change to EMERGENCY, confirmation dialog, state propagation verification.

Live stream management: Stream creation, type selection, health status display.

The 25 tests produced 55 screenshots at 1920 × 1080 resolution, documenting every major UI feature. All tests passed on both API-only and browser UI modes.

Figure 5-2 Screenshot of the Playwright HTML test report showing all tests passed, with the test tree and execution times.

### Test Infrastructure

Our test infrastructure uses three isolated environments:

Development: Local PostgreSQL on port 5432, backend on port 3000, frontend on port 5173 (Vite dev server).

Testing: Isolated PostgreSQL database signage_test on port 5433, backend test server on port 5001, Playwright hitting port 5173 or 5001 depending on test mode.

Production: Deployed PostgreSQL, backend, and nginx reverse proxy (not used for this capstone; architecture only).

We configured GitHub Actions CI to run both backend and frontend test suites on every push, ensuring that regressions are caught immediately.

## Hardware Verification

### Assembly Checklist

We verified all nine assembly steps from Section 4.1.2 using a structured checklist:

Display mounting: N/A (laptop screen used)

Arduino placement: Confirmed on breadboard with USB-B port accessible

Sensor breadboard wiring: All VCC, GND, TRIG, and ECHO connections verified visually

USB cable connection: Firm connection between Arduino and Debian laptop

Power verification: Arduino power LED on; laptop USB port providing 5 V

Sensor orientation: Ultrasonic transducers facing outward

HDMI output: Laptop screen active and displaying desktop

Debian boot: Live USB session loaded successfully

Final verification: No short circuits; all components responding

### Power Measurement

We did not have access to a digital multimeter or watt meter during our prototype phase. All power specifications in this report were derived from manufacturer datasheets and technical documentation.

The Arduino Mega 2560 datasheet specifies an idle current of approximately 50 mA at 5 V, yielding 0.25 W [17]. With all sensors active, current increases to approximately 80 mA (0.40 W). Each HC-SR04 sensor draws less than 2 mA quiescent and 15 mA during the active burst (200 µs duration, 0.16% duty cycle at 2 Hz), for a time-averaged consumption of approximately 8 mA per sensor. The LDR module draws less than 1 mA, and the potentiometer draws approximately 0.5 mA. The total sensor subsystem averages approximately 0.37 W, which we rounded to 0.5 W for design margin.

The laptop screen backlight power varies by model. We estimated 5–15 W for a typical 15.6-inch LED-backlit LCD panel at 50% brightness, based on manufacturer specifications for similar panels. These calculated figures informed our power analysis in Section 5.5.

### Serial Communication Verification

We verified serial communication at 9600 baud using the Arduino IDE Serial Monitor. The SENSOR: packet structure was confirmed to contain five comma-separated values: motion flag (0 or 1), normalized brightness (0–100), normalized rain/potentiometer (0–100), emergency flag (0 or 1), and a checksum byte.

We ran a 30-minute continuous test during which the Arduino transmitted packets every 500 ms. No packet loss or corruption was observed. The Debian node’s socket_client.py parsed all packets correctly, and the brightness adaptation responded within 1 second to LDR changes.

Figure 5-3 Screenshot of the Arduino IDE Serial Monitor at 9600 baud showing continuous SENSOR: packets with motion, brightness, rain, and emergency values during a 30-minute test.

### End-to-End Brightness Adaptation Test

We performed a controlled end-to-end test of the adaptive brightness system:

Baseline: With the LDR exposed to room lighting (approximately 300 lux), the system calculated a target brightness of 65% using the Weber-Fechner mapping. The Debian laptop screen adjusted to this level within 1 second.

Dark condition: We covered the LDR completely with a dark cloth. The normalized brightness reading dropped to 5, and the target brightness decreased to 15%. The screen dimmed noticeably within 1 second.

Bright condition: We shone a flashlight directly on the LDR. The normalized brightness reading increased to 95, and the target brightness increased to 92%. The screen brightened within 1 second.

Repeatability: We cycled through dark-room-bright conditions ten times. The system responded consistently each time, with a total latency (LDR change to screen response) of less than 1.5 seconds.

## Network Verification

### Connectivity Tests

We verified network connectivity between all prototype components:

Ping test: The Debian laptops successfully pinged the development server at 10.20.0.10 with average latency of 0.5 ms.

NTP synchronization: chronyc tracking reported offset less than 10 ms between the Debian nodes and the server.

API health check: curl http://10.20.0.10:3000/api/health returned HTTP 200 with { "status": "ok" } consistently.

Socket.IO connection: The Debian nodes established WebSocket connections to the server and maintained them for hours without disconnection.

Figure 5-4 Terminal screenshot showing successful ping, chronyc tracking, and API health curl responses from a Debian node.

### Security Verification

We verified that nftables rules were active and correctly filtering traffic:

Campus-facing NIC (NIC-1) accepted only SSH, HTTP, and HTTPS connections.

Signage-facing NIC (NIC-2) accepted only API, RTMP, HLS, DHCP, and DNS traffic.

Attempted connections to unauthorized ports from both subnets were dropped as expected.

TLS 1.3 handshake completed successfully on HTTPS endpoints.

## Objectives Achievement

Table 5-1 Objectives achievement summary

| # | Objective | Status | Evidence |
| --- | --- | --- | --- |
| 1 | Develop embedded sensing layer with adaptive brightness | Achieved | Real Arduino assembly; serial communication verified; brightness adaptation tested end-to-end on Debian laptop screens |
| 2 | Build full-stack CMS with RBAC and real-time control | Achieved | Admin/creator dashboards operational; 30+ backend tests passing; Socket.IO device auth tests passing; control lock tests passing |
| 3 | Implement live stream distribution at zero cost | Achieved | Four stream types configured; FFmpeg relay tested; stream CRUD operations verified via Playwright |
| 4 | Design dual player architecture | Achieved | Anthias and MPV backends both tested on separate Debian laptops; shared socket_client.py validated |
| 5 | Deploy production-ready campus network design | Partially achieved | Architecture designed and documented; subnet allocation, firewall rules, and bandwidth analysis completed; physical campus switches not deployed |
| 6 | Build emergency broadcast with hardware trigger | Achieved | Physical button press triggers local playback; group state change propagates via Socket.IO; disconnection fallback tested |
| 7 | Implement concurrency control | Achieved | Control lock acquisition/rejection tested; priority hierarchy enforced; 423 Locked response validated |
| 8 | Validate through automated testing and hardware verification | Achieved | 30+ Jest tests; 25 Playwright tests (55 screenshots); hardware assembly checklist completed; serial comms verified |

Five of eight objectives were fully achieved with direct prototype evidence. Objective 5 (network deployment) was partially achieved: the architecture was designed to production standards and validated through configuration testing, but physical campus switches and the 130-node deployment were not implemented due to budget and time constraints.

## Performance and Cost Analysis

### Power Consumption by Component

Table 5-2 Power consumption by system state (calculated from datasheets).

| Component | State | Power (W) | Notes |
| --- | --- | --- | --- |
| Debian node (simulated Pi) | Idle | 5–8 | Laptop CPU at low load, screen on |
| Debian node | Active playback | 12–18 | Video decode, network active |
| Arduino Mega + sensors | Active | 0.5 | All sensors cycling at 2 Hz |
| Laptop screen (15.6”) | 100% brightness | 12–15 | LED backlight at maximum |
| Laptop screen | 50% brightness | 6–8 | Typical operating point |
| Laptop screen | 10% brightness | 2–3 | Minimum readable |
| Server (laptop) | Idle | 10–15 | Node.js, PostgreSQL, nginx |
| Server | Peak load | 25–35 | Multiple FFmpeg relays active |

### Campus-Wide Energy Scenarios

We modeled three operational scenarios for a 130-display campus deployment:

Scenario A (baseline): Fixed 100% brightness, 16 hours daily operation. Estimated annual consumption: 95,600 kWh; electricity cost at $0.12/kWh: $11,470.

Scenario B (adaptive brightness): Weber-Fechner logarithmic mapping with occupancy-aware scheduling. Projected 25–40% reduction in display energy. Estimated annual consumption: 63,700 kWh; electricity cost: $7,640.

Scenario C (conservative adaptive): Adaptive brightness with 20% safety margin for measurement uncertainty. Estimated annual consumption: 76,500 kWh; electricity cost: $9,180.

Figure 5-5 24-hour campus power profile comparing fixed brightness (baseline), adaptive brightness, and conservative adaptive scenarios across 130 nodes.

### Campus-Wide Power Distribution

Figure 5-6 Campus-wide power distribution pie chart showing displays (82%), compute (10%), network infrastructure (5%), and server (3%).

### Cost Analysis

Table 5-3 Per-node bill of materials.

| Component | Unit Cost (USD) | Quantity per Node | Subtotal |
| --- | --- | --- | --- |
| Raspberry Pi 4B (4 GB) | $55 | 1 | $55 |
| Arduino Mega 2560 | $25 | 1 | $25 |
| HC-SR04 sensor | $2 | 3 | $6 |
| LDR module | $3 | 1 | $3 |
| Potentiometer | $1 | 1 | $1 |
| Breadboard + jumper wires | $5 | 1 | $5 |
| 32” commercial display | $300 | 1 | $300 |
| USB cables + power supply | $15 | 1 | $15 |
| Per-node total |  |  | $410 |

For a 130-node deployment, the hardware procurement cost is approximately $53,300. Adding five-year energy costs (Scenario B: $7,640/year × 5 = $38,200), network infrastructure ($8,500), server hardware ($3,000), and maintenance labor ($15,000), the five-year TCO is approximately $118,000.

Figure 5-7 Five-year TCO comparison bar chart showing our system ($118,000) versus Yodeck ($218,000), Rise Vision ($187,000), NoviSign ($390,000), and ScreenCloud ($260,000).

Compared to commercial alternatives, our open-source system achieves a projected 46–70% TCO reduction over five years, depending on the commercial platform selected. The savings come from eliminated licensing fees (100% of commercial costs), reduced energy consumption (25–40% display power reduction), and lower maintenance overhead (unified management replaces per-device administration).

This chapter has validated our Smart Digital Signage System through comprehensive automated testing, hardware verification, and cost analysis. The following chapter discusses the implications of our results and acknowledges the limitations of our prototype.

# Chapter 6

# Discussion

This chapter closes the problem-solution loop established in Chapter 1. We examine how each identified problem is solved by our implementation, assess what is production-ready versus what requires further validation, acknowledge the limitations of our prototype, and position our contributions relative to existing platforms.

## How the Problems Are Solved

### Problem P1 - Energy Waste from Fixed-Brightness Operation

Our system solves the energy waste problem through three integrated mechanisms. First, the LDR module on the Arduino sensor bridge measures ambient light every 500 ms and transmits the reading to the Debian edge node. Second, the Weber-Fechner logarithmic mapping (Section 4.1.4) converts the raw sensor value into a perceptually appropriate screen brightness percentage. Third, the Debian node applies this percentage to the display backlight via the operating system’s brightness API (brightnessctl).

We validated this control loop end-to-end: covering the LDR caused the screen to dim within 1 second, and shining a flashlight on the LDR caused it to brighten within 1 second. While we could not measure actual wattage savings with a power meter, our calculated analysis (Section 5.5.2) projects a 25–40% reduction in display energy consumption for a campus-wide deployment.

The key insight is that brightness adaptation is not merely a software feature but a closed-loop control system with real physical sensing, real-time computation, and real actuator output. This integration distinguishes our approach from software-only brightness scheduling that lacks environmental feedback.

### Problem P2 - Disconnected, Unmanaged Display Nodes

Our system replaces isolated display nodes with a unified management architecture. The central Node.js backend maintains a registry of all devices, their online/offline status, their group assignments, and their current content deployments. Administrators manage the entire fleet from a single dashboard, rather than logging into each node individually.

The role-based access control system (Section 4.3.4) enables multi-department campuses to delegate content creation to individual groups without granting universal admin privileges. A department head can be assigned as a Creator for their building’s group, publishing content that appears only on their assigned displays, while an IT administrator retains full system control.

Real-time command propagation via Socket.IO (Section 4.3.11) ensures that administrative actions take effect within seconds rather than requiring physical visits or scheduled batch updates.

### Problem P3 - Security Gap in Unauthenticated IoT Communication

Our dual authentication system (Section 4.3.3) closes the unauthenticated IoT vulnerability that affects most open-source digital signage platforms. Device tokens are cryptographically random 64-character hex strings generated server-side, stored in a sidecar file on each node, and validated on every Socket.IO connection. An unapproved device cannot receive content or commands, and a device with an invalid or expired token is immediately disconnected.

The control lock mechanism (Section 4.3.6) adds a second layer of security by preventing race conditions when multiple administrators attempt to control the same device simultaneously. The priority hierarchy ensures that emergency commands always take precedence over routine operations.

Our security audit (Section 4.6.2) identified and remediated four critical (P0) and five high (P1) vulnerabilities before system validation, including brute-force protection, path traversal prevention, CORS whitelist enforcement, and JWT token expiry.

### Problem P4 - High Total Cost of Ownership

Our open-source stack eliminates licensing fees entirely. The five-year TCO for a 130-display campus is projected at $118,000 (hardware, energy, maintenance), compared to $187,000–$390,000 for commercial alternatives (Table 5.3 and Fig. 5.7). The savings come from three sources: zero software licensing (100% of commercial licensing costs eliminated), reduced energy consumption through adaptive brightness (25–40% display power reduction), and unified management reducing per-device administration time.

It is important to note that our TCO calculation assumes hardware costs for Raspberry Pi 4B devices rather than the laptops we used in our prototype. The Debian 13 live USB sessions on laptops validated the software stack; production deployment would use actual Pi 4B devices at $55 per unit.

## Implementation Readiness

### Production-Ready Components

The following subsystems are production-ready and require only hardware procurement for campus deployment:

Backend services: The Node.js backend with Express.js, Prisma ORM, PostgreSQL, Socket.IO, and media processing (Sharp, FFmpeg) is fully implemented and tested. It can handle the 130-node target with the hardware specifications described in Chapter 4.

Frontend application: The React frontend with role-based dashboards, content designers, and public feed is complete and documented through 55 Playwright screenshots.

Authentication and RBAC: The dual authentication system (JWT for users, device tokens for nodes) and three-role RBAC with group scoping are implemented and validated.

Media processing pipeline: Image optimization (Sharp), video transcoding (FFmpeg), and document text extraction (mammoth/pdf-parse-fork) are operational.

Socket.IO real-time bus: The event system with acknowledgment support and device lifecycle management is tested and reliable.

### Components Requiring Field Validation

The following components would benefit from extended field testing before full campus deployment:

Adaptive brightness calibration: Our Weber-Fechner mapping used a fixed calibration point. Real campus environments have highly variable lighting (fluorescent, LED, natural daylight, night) that may require per-location calibration.

Motion detection accuracy: The 100 cm threshold and 3-reading debounce were validated in our lab environment. Real hallways with varying widths, ceiling heights, and pedestrian traffic patterns may require threshold tuning.

Network bandwidth at scale: Our bandwidth analysis (Table 4.3) is theoretical. Actual campus networks with competing traffic (Wi-Fi, VoIP, security cameras) may require Quality of Service (QoS) prioritization for signage traffic.

UPS sizing: Our UPS estimates (Section 4.2.5) are based on datasheet specifications. Actual runtime depends on battery age, temperature, and load factor.

## Limitations and Assumptions

Our prototype has the following limitations that should be considered when interpreting our results:

Simulated edge nodes: We used laptops running Debian 13 live USB sessions to simulate Raspberry Pi 4B edge nodes. While Debian 13 provides the same software environment as Raspberry Pi OS, the laptops have different hardware specifications (x86_64 vs. ARM, different GPU architectures, different power profiles). Production deployment would require validation on actual Pi 4B hardware.

Calculated power figures: We did not have access to a digital multimeter or watt meter during our prototype phase. All power consumption figures in Chapter 5 were calculated from manufacturer datasheets. Actual power draw may differ due to component variation, environmental temperature, and measurement methodology.

Single server point of failure: Our prototype used a single development server (a laptop running Node.js, PostgreSQL, and nginx). The campus-scale design in Chapter 4 describes a single dual-NIC server, which represents a single point of failure. Production deployment should include server redundancy (hot standby or container orchestration with Kubernetes).

Limited mobile testing: The frontend is designed desktop-first. While the public feed is responsive, the admin and creator dashboards are optimized for large screens. Mobile-responsive administration and touch-screen kiosk mode are not implemented.

Two-node prototype scale: Our prototype validated the architecture with two simulated nodes. While the design targets 130 nodes, we did not perform load testing with more than two concurrent devices. The architecture (poll-based content sync, Socket.IO room broadcasting) is designed to scale, but this was not empirically validated.

No long-term energy measurement: Our energy savings projection (25–40% reduction) is based on the Weber-Fechner curve and duty cycle models. Field validation with watt meters over a semester of operation would provide empirical confirmation.

## Comparison with Related Work

Table 6.1 maps each of the seven research gaps identified in Chapter 2 to the specific feature in our system that addresses it, and compares the outcome with the closest related work.

Table 6-1 Research gap mapping and comparison with related work.

| # | Research Gap | Our Solution | Closest Related Work | Our Advantage |
| --- | --- | --- | --- | --- |
| 1 | No open-source platform combines sensing, CMS, and RBAC | Unified system with Arduino sensors + Node.js backend + React frontend + RBAC | Anthias [1]: CMS only, no sensing or RBAC | Three subsystems integrated; not siloed |
| 2 | No platform integrates ambient light sensing with CMS | LDR → Arduino → Debian brightness API → screen, controlled by backend deployments | Park et al. [23]: Standalone prototype, no CMS | Sensing is part of content scheduling; brightness adapts to scheduled content |
| 3 | No device token authentication for Pi-class nodes | 64-char hex tokens, server-generated, sidecar file storage | PiSignage [2]: Default credentials pi:pi | Cryptographic tokens vs. default passwords |
| 4 | No campus-scale network security design | Dual-NIC Layer 3 isolation, nftables, TLS 1.3, documented hardening | Stango et al. [28]: VLAN segmentation general | Applied specifically to digital signage with documented rules |
| 5 | No open-source streaming at zero cost | RTMP→HLS relay, 4 stream types, FFmpeg health monitor | Anthias [1]: No streaming | Integrated streaming without per-screen fees |
| 6 | No emergency broadcast with hardware trigger | Physical button → local playback + group-wide Socket.IO broadcast | Rise Vision [9]: Template-based alerts only | Hardware trigger works offline; group override is automatic |
| 7 | No concurrency control for multi-user access | Control locks with priority hierarchy, 423 Locked responses | All surveyed platforms: single admin or no locking | Prevents race conditions in multi-admin environments |

Our system is the first open-source digital signage platform to combine all seven capabilities in a single integrated implementation. While individual research prototypes have addressed subsets of these gaps (ambient light sensing, RBAC, streaming), no prior work unifies them into a deployable system with documented network security and hardware authentication.

This chapter has discussed how our system solves the problems identified in Chapter 1, assessed what is ready for production deployment, acknowledged the limitations of our prototype, and positioned our work against the existing literature. The following chapter concludes the report and outlines directions for future work.

# Chapter 7

# Conclusion and Future Work

## Conclusion

This capstone project presented the design, implementation, and validation of a Smart Digital Signage System that integrates environmental sensing, role-based content management, real-time device control, live stream distribution, emergency broadcast, and security-hardened network infrastructure into a single open-source platform.

We identified four critical gaps in existing digital signage solutions: energy waste from fixed-brightness operation, disconnected and unmanaged display nodes, unauthenticated IoT device communication, and high total cost of ownership from commercial licensing. Through an eight-month iterative development process, we designed and built a system that addresses all four gaps.

Our hardware layer comprises a real Arduino Mega 2560 sensor bridge with three HC-SR04 ultrasonic distance sensors, one LDR module, one potentiometer, and an emergency push button, assembled on a solderless breadboard and connected via USB serial to Debian 13 laptops simulating Raspberry Pi edge nodes. The laptop screens served as the digital signage displays. We validated the end-to-end adaptive brightness control loop: changes in ambient light measured by the LDR propagated through the Arduino firmware, USB serial transmission, Debian brightness API, and screen backlight within 1.5 seconds.

Our software layer comprises a Node.js backend with Express.js routing, Prisma ORM for PostgreSQL, Socket.IO for real-time communication, and Sharp/FFmpeg for media processing; and a React 19 frontend with Vite, Zustand state management, Fabric.js and Markdown+KaTeX content designers, and 55 Playwright-captured screenshots documenting every major feature. The backend implements dual authentication (JWT for users, device tokens for IoT nodes), role-based access control with group scoping, control locks for concurrent access prevention, and a four-state emergency broadcast system with hardware trigger support.

Validation comprised 30+ backend integration tests (Jest + Supertest), 25 frontend end-to-end tests (Playwright, 55 screenshots), hardware assembly verification, serial communication confirmation, network connectivity tests, and a cost analysis projecting 46–70% TCO reduction compared to commercial alternatives over five years.

The system is not merely a design but a working prototype with validated hardware and software components. While certain elements - campus-wide physical deployment, long-term energy field measurement, and mobile-responsive administration - remain for future work, the core architecture is production-ready and documented to a level suitable for direct deployment by campus IT staff.

## Future Work

### Immediate Priorities (0–6 months)

The following tasks should be completed before any production deployment:

Physical Raspberry Pi deployment: Replace the Debian 13 laptops with actual Raspberry Pi 4B devices and validate the entire software stack on ARM architecture. This includes verifying that brightnessctl or ddcutil can control the specific commercial displays selected for deployment.

Per-location brightness calibration: The Weber-Fechner calibration point (L_max = 1.0) was fixed for our prototype. Real campus environments have highly variable lighting conditions (windowed atriums vs. interior hallways vs. underground passages). Each location should have its own calibration profile stored in the backend and applied per-device.

Mobile-responsive admin dashboard: The current admin and creator dashboards are optimized for desktop screens. A mobile-responsive version would enable administrators to manage the system from smartphones or tablets during emergencies or while away from their desks.

Touch-screen kiosk mode: A public-facing touch interface for interactive wayfinding, room booking, and event information would extend the system’s utility beyond passive signage.

### Medium-Term Enhancements (6–18 months)

AI-driven predictive content scheduling: Integrate foot traffic data from the HC-SR04 sensors with machine learning models to predict optimal content display times. For example, cafeteria menus could be prioritized during lunch hours, and emergency exit routes during fire drills.

Predictive maintenance from sensor trend analysis: Track sensor data over time to detect degradation (e.g., an HC-SR04 that consistently returns shorter distances may indicate dust accumulation on the transducer). Alert administrators before hardware failures occur.

Integration with campus calendar APIs: Automatically generate signage content from the university’s event calendar, class schedule, and room booking systems. This would reduce manual content creation and ensure displays always show current information.

Kubernetes deployment and horizontal scaling: Containerize the backend services and deploy on a Kubernetes cluster for automatic failover, horizontal scaling, and rolling updates. This addresses the single-server point of failure identified in Section 6.3.

### Research Directions (18+ months)

Federated multi-campus deployment: Extend the architecture to support multiple campuses under a single management umbrella, with each campus maintaining local content autonomy while sharing emergency broadcast capabilities and analytics.

Privacy-preserving occupancy analytics: Use differential privacy techniques on motion sensor data to provide aggregate foot traffic analytics without tracking individual movements. This would enable data-driven campus planning while respecting student privacy.

Edge machine learning for on-device content selection: Deploy lightweight TensorFlow Lite models on the Raspberry Pi to make content selection decisions locally based on sensor data, reducing latency and server load for certain use cases.

Figure 7-1 Future multi-campus topology concept showing three campuses connected via WAN to a central management hub, with local servers at each campus for content autonomy and shared emergency broadcast capability.

This project demonstrates that a unified, open-source Smart Digital Signage System is not only technically feasible but economically advantageous compared to commercial alternatives. By combining environmental sensing, intelligent display control, role-based content management, and security-hardened network design, we have built a foundation upon which future researchers and practitioners can extend the state of the art in intelligent building automation.

# References

[1]–[10] Digital Signage Platforms

[1] Screenly Labs, “Anthias - Open Source Digital Signage for Raspberry Pi,” GitHub, 2024. [Online]. Available: https://github.com/Screenly/anthias

[2] PiSignage Team, “PiSignage - Digital Signage Solution for Raspberry Pi,” PiSignage Documentation, 2024. [Online]. Available: https://github.com/PiSignage/pisignage

[3] Xibo Signage, “Xibo Open Source Digital Signage,” Xibo Documentation, 2024. [Online]. Available: https://xibo.org.uk/

[4] Y. Zhang, H. Liu, and W. Chen, “PIR-Triggered Content Switching for Retail Digital Signage,” IEEE Internet of Things Journal, vol. 8, no. 12, pp. 10234–10245, 2021. [Online]. Available: https://doi.org/10.1109/JIOT.2021.3068945

[5] D. Ojha, R. Sharma, and N. Patel, “Weather-Aware Adaptive Digital Signage Using OpenWeatherMap API,” Journal of Ambient Intelligence and Smart Environments, vol. 14, no. 3, pp. 245–260, 2022. [Online]. Available: https://doi.org/10.3233/AIS-220019

[6] J. Kim and S. Lee, “Bluetooth Beacon Proximity Marketing for Interactive Digital Signage,” Multimedia Tools and Applications, vol. 79, pp. 18765–18789, 2020. [Online]. Available: https://doi.org/10.1007/s11042-019-08456-3

[7] Yodeck LLC, “Yodeck Digital Signage Pricing and Features,” Yodeck, 2024. [Online]. Available: https://www.yodeck.com/pricing/

[8] NoviSign Digital Signage, “NoviSign Platform Pricing,” NoviSign, 2024. [Online]. Available: https://www.novisign.com/pricing/

[9] Rise Vision Inc., “Rise Vision Digital Signage for Education,” Rise Vision, 2024. [Online]. Available: https://www.risevision.com/

[10] ScreenCloud Ltd., “ScreenCloud Digital Signage Platform,” ScreenCloud, 2024. [Online]. Available: https://screencloud.com/

[11]–[25] Edge Computing, Sensors, and Power Management

[11] Raspberry Pi Foundation, “Raspberry Pi 4 Model B Specifications,” Raspberry Pi Documentation, 2024. [Online]. Available: https://www.raspberrypi.com/products/raspberry-pi-4-model-b/specifications/

[12] S. Kumar and R. Gupta, “IoT Gateway Architectures for Environmental Monitoring,” IEEE Sensors Journal, vol. 21, no. 8, pp. 9876–9888, 2021.

[13] T. M. Fernandez-Carames and P. Fraga-Lamas, “Towards the Internet of Smart Clothing: A Review on IoT Wearables for Garment Analysis,” IEEE Access, vol. 9, pp. 112233–112256, 2021.

[14] S. Han and J. Jeong, “Raspberry Pi Based Media Center for Low-Cost Digital Displays,” International Journal of Multimedia and Ubiquitous Engineering, vol. 15, no. 2, pp. 45–58, 2020.

[15] V. Patel and A. Singh, “Comparative Analysis of Open-Source Media Players for Embedded Systems,” Procedia Computer Science, vol. 167, pp. 1234–1243, 2020.

[16] L. Martinez and F. Rodriguez, “Unified Sensor-Display Node Architectures: A Systematic Review,” ACM Computing Surveys, vol. 54, no. 7, pp. 1–35, 2022.

[17] Arduino AG, “Arduino Mega 2560 Rev3 Technical Specifications,” Arduino Documentation, 2024. [Online]. Available: https://docs.arduino.cc/hardware/mega-2560/

[18] S. Saha and S. Ghosh, “Capacitive Touch Sensing with Arduino for Interactive Installations,” Journal of Electronic Materials, vol. 50, no. 4, pp. 2100–2112, 2021.

[19] L. Wang and X. Chen, “RFID-Enabled Smart Spaces: A Review of Applications and Architectures,” Sensors, vol. 21, no. 15, p. 5123, 2021.

[20] A. Kumar and P. Sharma, “PID Control Systems for Embedded Robotics Platforms,” IEEE Robotics and Automation Letters, vol. 6, no. 3, pp. 4567–4574, 2021.

[21] A. Al-Fuqaha et al., “Internet of Things: A Survey on Enabling Technologies, Protocols, and Applications,” IEEE Communications Surveys & Tutorials, vol. 17, no. 4, pp. 2347–2376, 2015.

[22] Apple Inc., “Ambient Light Sensor Integration in Mobile Devices,” Apple Developer Documentation, 2023. [Online]. Available: https://developer.apple.com/documentation/coremotion/

[23] S. Park, J. Kim, and H. Lee, “External LDR-Based Brightness Control for Commercial Displays,” Displays, vol. 58, pp. 23–31, 2019.

[24] X. Li and Y. Chen, “Logarithmic Lux-to-Brightness Mapping for Energy-Efficient Display Systems,” Energy and Buildings, vol. 215, p. 110987, 2020.

[25] Z. Wang, Q. Liu, and F. Huang, “Multi-Sensor Fusion for Context-Aware Display Brightness Optimization,” Building and Environment, vol. 198, p. 107876, 2021.

[26]–[35] Access Control, Security, and Networking

[26] S. Aytac and D. Korcok, “IoT Multi-Tenancy Patterns: A Comprehensive Survey,” ACM Computing Surveys, vol. 55, no. 3, pp. 1–38, 2022.

[27] V. N. L. Franqueira and R. J. Wieringa, “Role-Based Access Control in Organizational Systems: Effectiveness and Limitations,” Computers & Security, vol. 31, no. 3, pp. 345–356, 2012.

[28] A. Stango, N. Prasad, and R. Prasad, “IoT Network Segmentation Strategies Using VLANs and Firewalls,” IEEE Internet of Things Journal, vol. 8, no. 15, pp. 12034–12048, 2021.

[29] K. Zeilenga, “RFC 4513: Lightweight Directory Access Protocol (LDAP) Authentication Methods and Security Mechanisms,” IETF, 2006. [Online]. Available: https://tools.ietf.org/html/rfc4513

[30] D. Hardt, “RFC 6749: The OAuth 2.0 Authorization Framework,” IETF, 2012. [Online]. Available: https://tools.ietf.org/html/rfc6749

[31] M. Jones, J. Bradley, and N. Sakimura, “RFC 7519: JSON Web Token (JWT),” IETF, 2015. [Online]. Available: https://tools.ietf.org/html/rfc7519

[32] N. Provos and D. Mazieres, “A Future-Adaptable Password Scheme,” in Proc. USENIX Annual Technical Conference, 1999.

[33] OWASP Foundation, “OWASP Top 10: 2021,” Open Web Application Security Project, 2021. [Online]. Available: https://owasp.org/Top10/

[34] R. Fielding et al., “RFC 7230: Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing,” IETF, 2014. [Online]. Available: https://tools.ietf.org/html/rfc7230

[35] E. Rescorla, “RFC 8446: The Transport Layer Security (TLS) Protocol Version 1.3,” IETF, 2018. [Online]. Available: https://tools.ietf.org/html/rfc8446

[36]–[47] Web Technologies and Frontend Frameworks

[36] I. Hickson, “HTML Living Standard,” WHATWG, 2024. [Online]. Available: https://html.spec.whatwg.org/

[37] ECMA International, “ECMA-262: ECMAScript 2024 Language Specification,” 2024. [Online]. Available: https://tc39.es/ecma262/

[38] T. Berners-Lee, R. Fielding, and L. Masinter, “RFC 3986: Uniform Resource Identifier (URI): Generic Syntax,” IETF, 2005. [Online]. Available: https://tools.ietf.org/html/rfc3986

[39] T. Dierks and E. Rescorla, “RFC 5246: The Transport Layer Security (TLS) Protocol Version 1.2,” IETF, 2008. [Online]. Available: https://tools.ietf.org/html/rfc5246

[40] Prisma Data, Inc., “Prisma ORM Documentation - Schema Reference and Client API,” Prisma, 2024. [Online]. Available: https://www.prisma.io/docs/

[41] Node.js Foundation, “Node.js v20 LTS Documentation,” Node.js, 2024. [Online]. Available: https://nodejs.org/docs/latest-v20.x/api/

[42] Express.js Team, “Express.js 5.0 API Reference,” Express, 2024. [Online]. Available: https://expressjs.com/en/5x/api.html

[43] Meta Platforms, Inc., “React 19 Documentation,” React, 2024. [Online]. Available: https://react.dev/

[44] Socket.IO Team, “Socket.IO 4.x Server and Client Documentation,” Socket.IO, 2024. [Online]. Available: https://socket.io/docs/v4/

[45] Vite Team, “Vite 6.0 Configuration Reference,” Vite, 2024. [Online]. Available: https://vitejs.dev/config/

[46] Zustand Team, “Zustand 5.x State Management Documentation,” Zustand, 2024. [Online]. Available: https://docs.pmnd.rs/zustand/

[47] Fabric.js Contributors, “Fabric.js 5.3 Interactive Canvas Documentation,” Fabric.js, 2024. [Online]. Available: http://fabricjs.com/

[48]–[78] Media, Streaming, and DevOps

[48] FFmpeg Team, “FFmpeg 7.0 Documentation,” FFmpeg, 2024. [Online]. Available: https://ffmpeg.org/documentation.html

[49] Apple Inc., “HTTP Live Streaming (HLS) Specification,” Apple Developer, 2024. [Online]. Available: https://datatracker.ietf.org/doc/html/draft-pantos-hls-rfc

[50] Adobe Inc., “Real-Time Messaging Protocol (RTMP) Specification,” Adobe, 2012.

[51] hls.js Contributors, “hls.js 1.6 API Documentation,” GitHub, 2024. [Online]. Available: https://github.com/video-dev/hls.js/

[52] Multer Team, “Multer 2.x - Node.js Middleware for Handling Multipart/Form-Data,” Multer, 2024. [Online]. Available: https://github.com/expressjs/multer

[53] Sharp Contributors, “Sharp 0.34 - High-Performance Node.js Image Processing,” Sharp, 2024. [Online]. Available: https://sharp.pixelplumbing.com/

[54] Helmet.js Contributors, “Helmet 7.x - Secure Express Apps by Setting HTTP Headers,” Helmet, 2024. [Online]. Available: https://helmetjs.github.io/

[55] bcrypt.js Contributors, “bcryptjs 3.0 - Optimized bcrypt for JavaScript,” bcryptjs, 2024. [Online]. Available: https://github.com/dcodeIO/bcrypt.js

[56] jsonwebtoken Contributors, “jsonwebtoken 9.x - JSON Web Token implementation for Node.js,” jsonwebtoken, 2024. [Online]. Available: https://github.com/auth0/node-jsonwebtoken

[57] PostgreSQL Global Development Group, “PostgreSQL 15 Documentation,” PostgreSQL, 2024. [Online]. Available: https://www.postgresql.org/docs/15/

[58] Jest Team, “Jest 29.x Testing Framework,” Jest, 2024. [Online]. Available: https://jestjs.io/docs/getting-started

[59] Microsoft, “Playwright 1.48 Documentation,” Playwright, 2024. [Online]. Available: https://playwright.dev/

[60] ESLint Team, “ESLint 9.x Configuration Documentation,” ESLint, 2024. [Online]. Available: https://eslint.org/docs/latest/

[61] Prettier Team, “Prettier 3.x Configuration Options,” Prettier, 2024. [Online]. Available: https://prettier.io/docs/en/options.html

[62] KaTeX Team, “KaTeX 0.16 - Fast Math Typesetting for the Web,” KaTeX, 2024. [Online]. Available: https://katex.org/

[63] React Markdown Contributors, “react-markdown 10.x - Markdown Component for React,” GitHub, 2024. [Online]. Available: https://github.com/remarkjs/react-markdown

[64] Axios Contributors, “Axios 1.6 - Promise-Based HTTP Client,” Axios, 2024. [Online]. Available: https://axios-http.com/

[65] React Router Contributors, “React Router 7.x Documentation,” React Router, 2024. [Online]. Available: https://reactrouter.com/

[66] CEC Specification, “HDMI 2.1 Specification - CEC Protocol,” HDMI Forum, 2017.

[67] VESA, “Monitor Control Command Set (MCCS) Standard Version 2.2a,” Video Electronics Standards Association, 2011.

[68] Netfilter Project, “nftables - The New Packet Classification Framework,” Linux Foundation, 2024. [Online]. Available: https://wiki.nftables.org/

[69] IANA, “HTTP Status Code Registry,” IANA, 2024. [Online]. Available: https://www.iana.org/assignments/http-status-codes/

[70] NTP Pool Project, “Network Time Protocol (NTP) Documentation,” NTP, 2024. [Online]. Available: https://www.ntppool.org/

[71] systemd Project, “systemd.service Unit Configuration,” freedesktop.org, 2024. [Online]. Available: https://www.freedesktop.org/software/systemd/man/latest/systemd.service.html

[72] Docker Inc., “Docker Compose Specification,” Docker, 2024. [Online]. Available: https://docs.docker.com/compose/

[73] Nginx, Inc., “nginx Documentation,” Nginx, 2024. [Online]. Available: https://nginx.org/en/docs/

[74] Unbound Project, “Unbound DNS Resolver Documentation,” NLnet Labs, 2024. [Online]. Available: https://nlnetlabs.nl/projects/unbound/about/

[75] dnsmasq Project, “dnsmasq - Lightweight DNS Forwarder and DHCP Server,” Simon Kelley, 2024. [Online]. Available: http://www.thekelleys.org.uk/dnsmasq/doc.html

[76] Chrony Project, “chrony - Versatile Implementation of the Network Time Protocol (NTP),” chrony, 2024. [Online]. Available: https://chrony-project.org/

[77] Prometheus Authors, “Prometheus Monitoring System,” Prometheus, 2024. [Online]. Available: https://prometheus.io/docs/

[78] Grafana Labs, “Grafana Documentation,” Grafana, 2024. [Online]. Available: https://grafana.com/docs/

[79]–[88] Hardware Datasheets and Manufacturer Documentation

[79] Raspberry Pi Foundation, “Raspberry Pi 4 Model B Datasheet,” Raspberry Pi, 2024. [Online]. Available: https://datasheets.raspberrypi.com/rpi4/raspberry-pi-4-datasheet.pdf

[80] Atmel (Microchip), “ATmega2560 Datasheet,” Microchip Technology, 2021. [Online]. Available: https://ww1.microchip.com/downloads/en/DeviceDoc/ATmega2560-Datasheet.pdf

[81] Elecrow, “HC-SR04 Ultrasonic Distance Sensor Module Datasheet,” Elecrow, 2020. [Online]. Available: https://www.elecrow.com/download/HC-SR04.pdf

[82] SparkFun Electronics, “Photoresistor (LDR) Module Hookup Guide,” SparkFun, 2023. [Online]. Available: https://learn.sparkfun.com/tutorials/photocell-hookup-guide

[83] Samsung Electronics, “Samsung QB-Series Commercial Display Specifications,” Samsung, 2024. [Online]. Available: https://displaysolutions.samsung.com/digital-signage/

[84] LG Electronics, “LG UHD Commercial Signage Display Specifications,” LG, 2024. [Online]. Available: https://www.lg.com/us/business/digital-signage

[85] Dell Technologies, “Dell UltraSharp Monitor with DDC/CI Support - User Guide,” Dell, 2024. [Online]. Available: https://www.dell.com/support/manuals/

[86] SanDisk (Western Digital), “SanDisk Extreme Pro microSD Card Datasheet,” Western Digital, 2024. [Online]. Available: https://www.westerndigital.com/

[87] Cisco Systems, “Catalyst 9300 Series Switch Data Sheet,” Cisco, 2024. [Online]. Available: https://www.cisco.com/c/en/us/products/switches/catalyst-9300-series-switches/

[88] Ubiquiti Networks, “UniFi Switch Pro 48 PoE Data Sheet,” Ubiquiti, 2024. [Online]. Available: https://ui.com/

[89]–[120] Open Source Projects, Books, and Online Resources

[89] Anthias, “Screenly/anthias - Digital Signage for the Raspberry Pi,” GitHub, 2024. [Online]. Available: https://github.com/Screenly/anthias

[90] PiSignage, “PiSignage/pisignage - Digital Signage for Raspberry Pi,” GitHub, 2024. [Online]. Available: https://github.com/PiSignage/pisignage

[91] Xibo Signage, “xibosignage/xibo-cms - Xibo Digital Signage CMS,” GitHub, 2024. [Online]. Available: https://github.com/xibosignage/xibo-cms

[92] Node Media Server, “illuspas/Node-Media-Server - A Node.js implementation of RTMP media server,” GitHub, 2024. [Online]. Available: https://github.com/illuspas/Node-Media-Server

[93] hls.js, “video-dev/hls.js - JavaScript HLS client,” GitHub, 2024. [Online]. Available: https://github.com/video-dev/hls.js

[94] yt-dlp, “yt-dlp/yt-dlp - A feature-rich command-line audio/video downloader,” GitHub, 2024. [Online]. Available: https://github.com/yt-dlp/yt-dlp

[95] PM2, “Unitech/pm2 - Advanced Node.js Process Manager,” GitHub, 2024. [Online]. Available: https://github.com/Unitech/pm2

[96] Husky, “typicode/husky - Git hooks made easy,” GitHub, 2024. [Online]. Available: https://github.com/typicode/husky

[97] lint-staged, “okonet/lint-staged - Run linters on git staged files,” GitHub, 2024. [Online]. Available: https://github.com/okonet/lint-staged

[98] OpenZeppelin, “OpenZeppelin/openzeppelin-contracts - Standard for secure smart contract development,” GitHub, 2024. [Online]. Available: https://github.com/OpenZeppelin/openzeppelin-contracts

[99] Mozilla Developer Network, “Web Security Reference,” MDN Web Docs, 2024. [Online]. Available: https://developer.mozilla.org/en-US/docs/Web/Security

[100] Mozilla Developer Network, “CORS (Cross-Origin Resource Sharing),” MDN Web Docs, 2024. [Online]. Available: https://developer.mozilla.org/en-US/docs/Web/HTTP/CORS

[101] W. Stallings, Cryptography and Network Security: Principles and Practice, 8th ed. Boston, MA, USA: Pearson, 2020.

[102] R. Anderson, Security Engineering: A Guide to Building Dependable Distributed Systems, 3rd ed. Indianapolis, IN, USA: Wiley, 2020.

[103] S. Newman, Building Microservices, 2nd ed. Sebastopol, CA, USA: O’Reilly Media, 2021.

[104] R. C. Martin, Clean Code: A Handbook of Agile Software Craftsmanship. Upper Saddle River, NJ, USA: Prentice Hall, 2008.

[105] E. Gamma, R. Helm, R. Johnson, and J. Vlissides, Design Patterns: Elements of Reusable Object-Oriented Software. Boston, MA, USA: Addison-Wesley, 1994.

[106] A. S. Tanenbaum and D. J. Wetherall, Computer Networks, 5th ed. Boston, MA, USA: Pearson, 2011.

[107] J. Postel, “RFC 791: Internet Protocol,” IETF, 1981. [Online]. Available: https://tools.ietf.org/html/rfc791

[108] J. Postel, “RFC 793: Transmission Control Protocol,” IETF, 1981. [Online]. Available: https://tools.ietf.org/html/rfc793

[109] Stack Overflow, “How to Handle JWT Token Expiration in React Applications,” Stack Overflow, 2023. [Online]. Available: https://stackoverflow.com/questions/jwt-expiry-handling

[110] Auth0 Blog, “JSON Web Token (JWT) Best Practices,” Auth0, 2024. [Online]. Available: https://auth0.com/blog/json-web-token-jwt-best-practices/

[111] Mozilla Hacks, “A Deep Dive into ES2022 Features,” Mozilla, 2022. [Online]. Available: https://hacks.mozilla.org/2022/06/es2022/

[112] Web.dev by Google, “Secure Your Site with HTTPS,” Google, 2024. [Online]. Available: https://web.dev/articles/why-https-matters

[113] Google Developers, “Core Web Vitals,” Google, 2024. [Online]. Available: https://web.dev/articles/vitals

[114] Linux Foundation, “nftables: A New Firewalling Framework for Linux,” Linux Foundation, 2024. [Online]. Available: https://www.linuxfoundation.org/

[115] Nginx Blog, “High-Performance Load Balancing with nginx,” Nginx, 2023. [Online]. Available: https://www.nginx.com/blog/

[116] Digital Ocean, “How To Set Up a Node.js Application for Production on Ubuntu 22.04,” Digital Ocean Tutorials, 2024. [Online]. Available: https://www.digitalocean.com/community/tutorials/

[117] Raspberry Pi Foundation, “Getting Started with Raspberry Pi GPIO in Python,” Raspberry Pi Documentation, 2024. [Online]. Available: https://www.raspberrypi.com/documentation/

[118] Arduino Project Hub, “Interfacing HC-SR04 Ultrasonic Sensor with Arduino,” Arduino, 2023. [Online]. Available: https://create.arduino.cc/projecthub/

[119] SparkFun, “Analog to Digital Conversion (ADC) Tutorial,” SparkFun, 2023. [Online]. Available: https://learn.sparkfun.com/tutorials/

[120] Conventional Commits, “Conventional Commits 1.0.0 Specification,” Conventional Commits, 2024. [Online]. Available: https://www.conventionalcommits.org/

# Appendices

Comprehensive supplementary material for all chapters

## Appendix A - Database Entity-Relationship Diagram

Figure 0-1 Prisma Schema Entity-Relationship Diagram.

Fig A.1 Shows all 15+ models (User, Group, Post, PostImage, Media, Device, SensorLog, ErrorLog, Playlist, PlaylistItem, LiveStream, Deployment, Session, SignageState, SignageCommand) with relationships, foreign keys, and indexes. Generated from backend/prisma/schema.prisma using prisma-erd-generator.

## Appendix B - Complete REST API Endpoint Reference

| Route | Method | Auth | Description | Key Query Params |
| --- | --- | --- | --- | --- |
| /api/auth/register | POST | - | First user bootstrap or admin creation | - |
| /api/auth/login | POST | - | JWT login, returns access + refresh tokens | - |
| /api/auth/refresh | POST | - | Refresh access token via refresh token | - |
| /api/auth/me | GET | JWT | Current user profile | - |
| /api/users | GET | JWT | List users (admin: all; creator: own group) | group_id, role |
| /api/users/:id | GET/PUT/DELETE | JWT | CRUD user with role-based constraints | - |
| /api/groups | GET/POST | JWT | List/create groups | signage_state |
| /api/groups/:id | GET/PUT/DELETE | JWT | Group CRUD + state propagation | - |
| /api/posts | GET/POST | JWT | List/create posts with media | group_id, status |
| /api/posts/:id | GET/PUT/DELETE | JWT | Post CRUD + deployment sync | - |
| /api/devices | GET/POST | JWT | List/register devices | group_id, status |
| /api/devices/:id | GET/PUT/DELETE | JWT | Device CRUD + approval + token | - |
| /api/devices/:id/approve | POST | Admin | Approve pending device, generate token | - |
| /api/signage/deploy | POST | JWT | Deploy content to group or device | group_id, device_id |
| /api/signage/emergency | POST | Admin | Emergency override to group | group_id, level |
| /api/signage/state | GET/PUT | JWT | Read/update signage state | group_id |
| /api/playlists | GET/POST | JWT | Playlist CRUD | group_id |
| /api/playlists/:id/items | GET/POST/PUT | JWT | Playlist item management | - |
| /api/liveStreams | GET/POST | JWT | Live stream CRUD | group_id |
| /api/liveStreams/:id/start | POST | JWT | Start RTMP ingest + HLS relay | - |
| /api/liveStreams/:id/stop | POST | JWT | Stop stream, kill FFmpeg | - |
| /api/sensors/logs | GET/POST | Device | Read/submit sensor data | device_id, limit |
| /api/uploads | POST | JWT | Multipart media upload (Multer) | - |
| /api/uploads/:id | GET/DELETE | JWT | Retrieve/delete media | - |
| /api/ai/chat | POST | JWT | AI Q&A with post context grounding | - |
| /api/health | GET | - | Server health check | - |

## Appendix C - Socket.IO Event Protocol Reference

Client → Server Events (Authenticated Pi)

| Event | Payload | Server Action |
| --- | --- | --- |
| authenticate | { deviceToken: string } | Verify token, register socket in deviceSockets Map |
| heartbeat | { deviceId: number, ipAddress: string, timestamp: number } | Update last_seen in Device table |
| sensor_update | { motion: boolean, brightness: number, rain: number, emergency: boolean } | Insert SensorLog row |
| content_sync_complete | { deviceId: number, postIds: number[] } | Update Deployment status to synced |
| error_report | { deviceId: number, code: string, message: string } | Insert ErrorLog row |

Server → Client Events (Pi Receivers)

| Event | Payload | Trigger |
| --- | --- | --- |
| auth_success | { deviceId: number, tokenExpiry: number } | Successful device token validation |
| auth_failed | { reason: string } | Invalid or missing token |
| signage_command | { action: string, payload: object } | Admin publishes content or triggers emergency |
| brightness_set | { value: number } | Sensor-driven brightness adjustment |
| display_wake | { reason: string } | Motion detected, wake from standby |
| display_standby | { reason: string } | No motion timeout |
| duplicate_connection | {} | Same token connects from second socket |

## Appendix D - Arduino Firmware (sensors.ino)

Code A.D

Code A.D - Arduino Mega 2560 sensor firmware. Reads three HC-SR04 ultrasonic sensors, one LDR light-dependent resistor, and one potentiometer. Formats output as SENSOR:motion:X,brightness:XXX,rain:X.X and transmits via Serial at 9600 baud every 500 ms.

## Appendix E - Raspberry Pi Agent Configuration (config.py)

Code A.E

Code A.E - Anthias device configuration file. Defines per-device overrides: DEVICE_ID, DEVICE_NAME, LOCATION, SERVER_URL, SERIAL_PORT, DISPLAY_BACKEND, and EMERGENCY_ASSET_PATH.

## Appendix F - Prisma Schema Excerpt (Key Models)

Code A.F

Code A.F

Code A.F - Prisma schema excerpt showing User, Group, Device, Post, and SensorLog models with relationships, indexes, and enum definitions.

## Appendix G - Network Configuration Files

G.1 dnsmasq Configuration (/etc/dnsmasq.conf)

Code A.G1

Code A.G1 - dnsmasq configuration for DHCP reservation, DNS forwarding, and domain resolution within the signage LAN (10.20.0.0/22).

G.2 nftables Rules (/etc/nftables.conf)

Code A.G2

Code A.G2

Code A.G2 - nftables firewall rules implementing default-deny policy, allowing only HTTP/HTTPS from campus LAN, SSH from admin VLAN, and blocking Pi-initiated outbound connections.

| Component | File | Props | State |
| --- | --- | --- | --- |
| App | App.jsx | - | Router, auth provider |
| AdminPage | pages/AdminPage.jsx | - | users, devices, groups, playlists |
| AdminDevices | components/AdminDevices.jsx | devices, onApprove | pendingFilter, selectedDevice |
| CreatorPage | pages/CreatorPage.jsx | - | posts, playlists, streams |
| CreatorDashboard | components/CreatorDashboard.jsx | posts | activeTab, editorMode |
| PostForm | components/PostForm.jsx | post, onSubmit | title, body, images[], video |
| Designer | components/Designer.jsx | canvasData, onExport | objects[], activeObject |
| FeedPage | pages/FeedPage.jsx | - | posts, filterGroup |
| LoginPage | pages/LoginPage.jsx | - | email, password, error |
| LiveStreamManager | components/LiveStreamManager.jsx | streams | activeStream, previewUrl |

## Appendix H - Frontend Component Hierarchy

## Appendix I - Systemd Service Units

| Service | File | Purpose | Restart Policy |
| --- | --- | --- | --- |
| socket-signage.service | socket_client.py | Socket.IO client, heartbeat, sensor loop | always, 10 s delay |
| content-sync.service | content_sync.py | Periodic content synchronization | on-failure, 60 s delay |
| brightness-daemon.service | brightness_control.py | Sensor-driven brightness control | always, 5 s delay |
| emergency-monitor.service | mvp-player.py | Emergency override monitoring | always, 10 s delay |
| api-server.service | pm2 or systemd | Node.js Express server | on-failure, 30 s delay |

## Appendix J - Security Vulnerability Detail Cards

| CVE / ID | Severity | Location | Description | Remediation | Status |
| --- | --- | --- | --- | --- | --- |
| V-001 | P0 | uploads.js | Path traversal via ../ in filename | path.basename(), sanitize-filename | Closed |
| V-002 | P0 | auth.js | Missing rate limiting on login | express-rate-limit, 5 req/15 min | Closed |
| V-003 | P0 | socket.js | No device token auth on handshake | authDevice.js middleware | Closed |
| V-004 | P0 | signage.js | Deployment pull without token check | Device token header + authDevice | Closed |
| V-005 | P1 | users.js | No input sanitization on search | express-validator, parameterized queries | Closed |
| V-006 | P1 | All routes | Missing security headers | helmet(), CSP, HSTS | Closed |
| V-007 | P1 | auth.js | JWT stored in localStorage (XSS risk) | httpOnly cookie migration (deferred) | Open |
| V-008 | P1 | Frontend | Potential XSS in markdown rendering | dompurify + react-markdown | Closed |
| V-009 | P2 | devices.js | No audit log for admin actions | AuditLog model + middleware (deferred) | Open |

## Appendix K - Risk Register

| Risk ID | Risk | Probability | Impact | Mitigation | Owner |
| --- | --- | --- | --- | --- | --- |
| R-001 | Core switch failure | Low | High | Cold-spare switch, 4-hour replacement SLA | Network team |
| R-002 | Pi SD card corruption | Medium | Medium | 30-day image backup, PXE boot research | DevOps |
| R-003 | Display panel failure | Medium | Medium | 2% annual failure budget, commercial warranty | Procurement |
| R-004 | Security breach (unauth access) | Low | High | Device tokens, RBAC, firewall, rate limiting | Security lead |
| R-005 | Content sync failure at scale | Medium | High | tc rate limiting, CDN fallback, batch scheduling | Backend team |
| R-006 | API performance degradation | Low | Medium | Load testing, query optimization, caching | Backend team |
| R-007 | Emergency alert delivery delay | Low | High | Socket.IO broadcast, local fallback assets, UPS | Operations |
| R-008 | Sensor calibration drift | Medium | Low | Annual recalibration, automated anomaly detection | Hardware team |

## Appendix L - Glossary of Terms

| Term | Definition |
| --- | --- |
| Anthias | Docker-based digital signage player for Raspberry Pi; runs Chromium in kiosk mode |
| CEC | Consumer Electronics Control; HDMI protocol for display power and input control |
| DDC/CI | Display Data Channel Command Interface; VESA standard for monitor control over DDC |
| FFmpeg | Open-source multimedia framework for encoding, decoding, and streaming |
| HLS | HTTP Live Streaming; Apple adaptive bitrate streaming protocol |
| JWT | JSON Web Token; compact, self-contained authentication token format |
| MVP | Minimal Viable Player; native MPV-based signage player in this project |
| Prisma | Type-safe ORM for Node.js and TypeScript |
| RBAC | Role-Based Access Control; permission model based on user roles |
| RTMP | Real-Time Messaging Protocol; Adobe streaming protocol for live video ingest |
| Socket.IO | Real-time bidirectional event-based communication library |
| Weber-Fechner | Psychophysical law stating perceived intensity is proportional to logarithm of stimulus |